Below is a structured preliminary design calculation for your rack and pinion.

- Given data

| Parameter | Symbol | Value |
| --- | --- | --- |
| Required rack travel | L | 30 to 50 mm |
| Rack axial load | F | 50 N |
| Available pinion diameter | d | 30 mm |
| Pinion rotation | N | 0.5 to 1 rev |
| Pressure angle | φ | 20° |
| Gear material |  | PLA |
| Required safety factor | SF | 2.5 |

For the calculation, I will use the maximum required travel of 50 mm and a 30 mm pitch diameter as the initial design basis.

- Fundamental rack and pinion relationship

For one complete revolution of the pinion:

L=πd

Where,

L=rack travel
d=pinion pitch diameter

For d=30mm:

L=π30
L=94.25 mm/rev

Therefore, the rack moves approximately 94.25 mm for one revolution.

- Rack travel for 0.5 to 1 revolution

The general equation is:

L=θ360∘πd

For 0.5 revolution:

L=180360π30
L=47.12 mm

For 1 revolution:

L=360360π30
L=94.25 mm

Therefore:

| Pinion rotation | Rack travel |
| --- | --- |
| 0.5 rev | 47.12 mm |
| 0.75 rev | 70.69 mm |
| 1 rev | 94.25 mm |

Your required 30 to 50 mm travel is therefore achievable within approximately 0.32 to 0.53 revolution.

- Required pinion rotation

Rearranging the equation:

θ=360Lπd

For 30 mm travel:

θ=36030π30
θ=114.59∘

For 50 mm travel:

θ=36050π30
θ=190.99∘

So:

θ=114.6∘ to 191.0∘

This corresponds to approximately:

0.319 to 0.531 revolution

- Selecting module and number of teeth

For a spur pinion:

d=mZ

where:

m= module in mm

Z= number of teeth

If we select:

m=2 mm

and

Z=15

then:

d=215
d=30 mm

This gives exactly your required 30 mm pitch diameter.

So the preliminary pinion specification is:

m=2 mm
Z=15
d=30 mm
ϕ=20∘

- Pinion outside diameter

For a standard spur gear:

do=mZ+2

Therefore:

do=215+2
do=34 mm

- Pinion root diameter

For a standard full-depth involute gear:

df=mZ-2.5

Therefore:

df=215-2.5
df=25 mm

Thus:

| Dimension | Value |
| --- | --- |
| Module | 2 mm |
| Teeth | 15 |
| Pitch diameter | 30 mm |
| Outside diameter | 34 mm |
| Root diameter | 25 mm |
| Pressure angle | 20° |

- Circular pitch

The circular pitch is:

p=πm

Therefore:

p=π2
p=6.283 mm

This is also the distance between corresponding teeth on the rack.

- Rack tooth pitch

For a rack:

p=πm

Therefore:

p=6.283 mm

The rack should therefore have a tooth pitch of approximately 6.28 mm.

- Rack travel per pinion tooth

One pinion tooth corresponds to one rack pitch.

Therefore:

Rack displacement per tooth=6.283 mm

For a 15-tooth pinion, one revolution gives:

156.283
94.25 mm

which agrees with:

L=πd

- Tangential force on pinion

The rack load is:

Ft=50 N

Therefore:

Ft=50 N

This is the primary force acting tangentially at the pinion pitch circle.

- Radial force

For a pressure angle of 20°:

Fr=Fttanϕ

Therefore:

Fr=50tan20∘
Fr=18.20 N

The resultant tooth force is:

Fn=Ftcosϕ
Fn=50cos20∘
Fn=53.21 N

So the gear tooth experiences approximately:

Ft=50 N
Fr=18.2 N
Fn=53.2 N

- Required pinion torque

The pitch radius is:

r=d2
r=302=15 mm

or:

r=0.015 m

Torque is:

T=Ftr

Therefore:

T=500.015
T=0.75 N\cdotpm

This is the ideal theoretical torque.

- Torque including safety factor

Using your required safety factor:

Tdesign=T×SF
Tdesign=0.752.5
Tdesign=1.875 N\cdotpm

Therefore, I would use approximately:

Tdesign≈1.88 N\cdotpm

as the preliminary design torque.

- Important point about PLA

For a PLA prototype, the 2.5 safety factor should not be treated as sufficient by itself.

PLA gears can fail through:

• Tooth bending

• Tooth wear

• Layer separation

• Creep

• Local tooth deformation

• Printing defects

Therefore, the next step should be a proper Lewis bending calculation.

- Lewis bending equation

The approximate bending stress can be calculated using:

σb=Ftb m Y

where:

σb=tooth bending stress
Ft=tangential force
b=face width
m=module
Y=Lewis form factor

For your design:

Ft=50 N
m=2 mm
Z=15

The face width bstill needs to be selected.

- Recommended preliminary geometry

For your prototype, I would start with:

| Parameter | Preliminary value |
| --- | --- |
| Module | 2 mm |
| Pinion teeth | 15 |
| Pitch diameter | 30 mm |
| Outside diameter | 34 mm |
| Root diameter | 25 mm |
| Pressure angle | 20° |
| Face width | 8 to 10 mm |
| Rack pitch | 6.283 mm |
| Rack travel | 30 to 50 mm |
| Rack load | 50 N |
| Required torque | 0.75 N·m |
| Design torque, SF 2.5 | 1.875 N·m |
| Material | PLA |

- Final mechanism relationship

Your complete kinematic relationship is:

L=θ360∘πmZ

Since:

m=2 mm

and:

Z=15

we obtain:

L=0.2618θ

where Lis in mm and θis in degrees.

Therefore:

30 mm travel→114.6∘
50 mm travel→191.0∘

So your 30 to 50 mm rack movement is achievable with approximately 115° to 191° pinion rotation.

One important correction: if your stated 30 mm "available pinion diameter" means the outside diameter, rather than the pitch diameter, then the above design changes. A 30 mm outside diameter with module 2 would require a different tooth count. For your mechanism, you should confirm which diameter the 30 mm refers to.

Yes. If 30 mm is the required outside diameter, we should calculate the pinion from:

dₒ = m(Z + 2)

For a standard 20° spur gear.

- Given parameters

| Parameter | Symbol | Value |
| --- | --- | --- |
| Outside diameter | dₒ | 30 mm |
| Rack load | F | 50 N |
| Pressure angle | φ | 20° |
| Safety factor | SF | 2.5 |
| Material |  | PLA |
| Required rack travel | L | 30 to 50 mm |
| Module | m | 1 or 1.5 mm |

- Option A, module = 1 mm

Number of teeth:

Z = dₒ/m − 2

Z = 30/1 − 2

Z = 28

Therefore:

Z = 28 teeth

Pitch diameter:

d = mZ

d = 1 × 28

d = 28 mm

Root diameter:

d_f = m(Z − 2.5)

d_f = 1(28 − 2.5)

d_f = 25.5 mm

So:

| Parameter | Value |
| --- | --- |
| Module | 1 mm |
| Teeth | 28 |
| Outside diameter | 30 mm |
| Pitch diameter | 28 mm |
| Root diameter | 25.5 mm |
| Pressure angle | 20° |

- Rack movement for module 1 mm

One revolution produces:

L = πd

L = π × 28

L = 87.96 mm/rev

For 30 mm travel:

θ = 360L/(πd)

θ = 360(30)/(π × 28)

θ = 122.78°

For 50 mm travel:

θ = 360(50)/(π × 28)

θ = 204.63°

Therefore:

30 mm travel → 122.8°

50 mm travel → 204.6°

- Torque for module 1 mm

Pitch radius:

r = d/2

r = 28/2 = 14 mm

r = 0.014 m

Required torque:

T = F r

T = 50 × 0.014

T = 0.70 N·m

With safety factor 2.5:

T_design = T × SF

T_design = 0.70 × 2.5

T_design = 1.75 N·m

Therefore:

T_design = 1.75 N·m

- Option B, module = 1.5 mm

Number of teeth:

Z = dₒ/m − 2

Z = 30/1.5 − 2

Z = 20 − 2

Z = 18

Therefore:

Z = 18 teeth

Pitch diameter:

d = mZ

d = 1.5 × 18

d = 27 mm

Root diameter:

d_f = m(Z − 2.5)

d_f = 1.5(18 − 2.5)

d_f = 23.25 mm

So:

| Parameter | Value |
| --- | --- |
| Module | 1.5 mm |
| Teeth | 18 |
| Outside diameter | 30 mm |
| Pitch diameter | 27 mm |
| Root diameter | 23.25 mm |
| Pressure angle | 20° |

- Rack movement for module 1.5 mm

One revolution:

L = πd

L = π × 27

L = 84.82 mm/rev

For 30 mm travel:

θ = 360(30)/(π × 27)

θ = 127.32°

For 50 mm travel:

θ = 360(50)/(π × 27)

θ = 212.21°

Therefore:

30 mm travel → 127.3°

50 mm travel → 212.2°

- Torque for module 1.5 mm

Pitch radius:

r = 27/2

r = 13.5 mm

r = 0.0135 m

Torque:

T = Fr

T = 50 × 0.0135

T = 0.675 N·m

With safety factor:

T_design = 0.675 × 2.5

T_design = 1.6875 N·m

Therefore:

T_design ≈ 1.69 N·m

- Force components

For both designs:

Tangential force:

F_t = 50 N

Radial force:

F_r = F_t tanφ

F_r = 50 tan20°

F_r = 18.20 N

Normal tooth force:

F_n = F_t/cosφ

F_n = 50/cos20°

F_n = 53.21 N

- Comparison

| Parameter | Module 1 | Module 1.5 |
| --- | --- | --- |
| Outside diameter | 30 mm | 30 mm |
| Teeth | 28 | 18 |
| Pitch diameter | 28 mm | 27 mm |
| Root diameter | 25.5 mm | 23.25 mm |
| Rack pitch | 3.142 mm | 4.712 mm |
| Rack travel/rev | 87.96 mm | 84.82 mm |
| Rotation for 30 mm | 122.8° | 127.3° |
| Rotation for 50 mm | 204.6° | 212.2° |
| Theoretical torque | 0.70 N·m | 0.675 N·m |
| Design torque, SF 2.5 | 1.75 N·m | 1.69 N·m |

- For your PLA prototype

I would use:

Module = 1.5 mm
Teeth = 18
Outside diameter = 30 mm
Pitch diameter = 27 mm
Pressure angle = 20°
Face width = approximately 8 to 10 mm
Rack module = 1.5 mm
Rack pitch = 4.712 mm

The 1.5 mm module gives fewer teeth and a larger tooth size than module 1 mm, which is useful for a 3D-printed PLA prototype carrying 50 N.

However, because your load is 50 N and the gear is PLA, the next calculation should be the tooth bending strength using the Lewis equation. That calculation will determine whether 1.5 mm module with an 8 to 10 mm face width actually meets your required safety factor of 2.5.

For your rack and pinion prototype, I would use a factor of safety of 2.5.

- Recommended value

| Application | Recommended FoS |
| --- | --- |
| Controlled laboratory prototype | 1.5 to 2.0 |
| 3D-printed PLA prototype | 2.5 to 3.0 |
| Variable or shock loading | 3.0 to 4.0 |
| Final safety-critical mechanism | Higher, based on applicable design standard |

- For your mechanism

Your conditions are:

• Material: PLA
• Load: 50 N
• 3D printed component
• Rack and pinion mechanism
• Possible variation in print quality and layer strength
• Prototype application

Therefore:

SF=2.5

is a reasonable design value.

For your calculations, use:

Fdesign=F×SF
Fdesign=50×2.5
Fdesign=125 N

Similarly, for the module 1.5 mm design:

Tdesign=0.675×2.5
Tdesign=1.69 N⋅m

So your gear should be checked against a design load of 125 N, rather than only 50 N.

Important: FoS = 2.5 does not automatically prove that the PLA gear is safe. You still need to check tooth bending strength, tooth root stress, wear, and the weaker 3D-printing direction.

Yes. The 300 N force should not simply be added to the 50 N force because you stated that they act in opposite directions.

For your mechanism, the load path is:

Rack ↔ Pinion ↔ Rack ↔ Spur Gear ↔ Sector Gear/Arm

Actually, based on your description, the rack meshes with the fixed pinion and also with the spur gear, while the spur gear meshes with the sector gear. I will calculate it as:

Rack → Spur gear → Sector gear/arm → 5 kg payload

and

Pinion → Rack → Spur gear → Sector gear/arm

I will use module 1.5 mm because you selected either 1 or 1.5 mm, and 1.5 mm gives a larger tooth size for your PLA prototype.

- Design assumptions

| Parameter | Symbol | Value |
| --- | --- | --- |
| Pinion module | m | 1.5 mm |
| Pinion outside diameter | dₒₚ | 30 mm |
| Pinion teeth | Zₚ | 18 |
| Pinion pitch diameter | dₚ | 27 mm |
| Pressure angle | φ | 20° |
| Rack direct force | Fᴅ | 50 N upward |
| Payload | M | 5 kg |
| Arm length | l | 300 mm |
| Sector pitch radius | rₛ | 49.5 mm |
| Sector teeth | Zₛ | 66 |
| Safety factor | SF | 2.5 |
| Gravity | g | 9.81 m/s² |

For the sector gear, I am selecting:

Zs=66

Therefore:

ds=mZs
ds=1.566
ds=99 mm

and:

rs=992
rs=49.5 mm

The 300 mm arm extends from the sector pivot. The sector pitch radius is therefore 49.5 mm, while the payload acts at 300 mm.

- Force produced by the 5 kg payload

Payload force:

W=Mg
W=59.81
W=49.05 N

So the payload produces:

W=49.05 N

- Torque required at the sector gear

The arm length is:

l=300 mm=0.3 m

Therefore:

Ts=Wl
Ts=49.050.3
Ts=14.715 N\cdotpm

This is the torque that the sector gear must provide to lift the 5 kg payload, assuming the arm is horizontal and ignoring friction.

- Tangential force at the sector gear

The sector pitch radius is:

rs=49.5 mm=0.0495 m

The required tangential force is:

Fs=Tsrs
Fs=14.7150.0495
Fs=297.27 N

This is where your approximately 300 N value comes from.

So your 300 N estimate is mechanically reasonable if the sector pitch radius is approximately 50 mm.

- Sector gear design force with FOS

You specified:

SF=2.5

Therefore:

Fs,design=FsSF
Fs,design=297.272.5
Fs,design=743.18 N

Therefore, the sector gear and spur gear mesh should be designed for approximately:

743 N

tangential design force.

- Direct force applied to the rack

You specified:

FD=50 N

acting upward.

The indirect force generated by the payload is:

FI=297.27 N

acting downward.

Because they act in opposite directions:

Frack=FI-FD
Frack=297.27-50
Frack=247.27 N downward

This is the net force that the pinion has to resist/transmit through the rack.

- Rack force with FOS

Frack,design=FrackSF
Frack,design=247.272.5
Frack,design=618.18 N

Therefore, for the net rack loading:

Frack,design≈618 N

- Important distinction between 247 N and 297 N

There are two different forces that you need to consider.

The rack-to-spur gear mesh carries the force generated by the payload:

Frack→spur=297.27 N

With FOS:

Frack→spur,design=743.18 N

The pinion-to-rack interaction sees the net force after the 50 N opposing force:

Frack→pinion=247.27 N

With FOS:

Frack→pinion,design=618.18 N

This distinction is important for your gear strength calculation.

- Pinion dimensions

For:

m=1.5 mm
Zp=18

Pitch diameter:

dp=mZp
dp=1.518
dp=27 mm

Outside diameter:

dop=mZp+2
dop=1.518+2
dop=30 mm

Root diameter:

dfp=mZp-2.5
dfp=1.518-2.5
dfp=23.25 mm

- Rack pitch

For a rack:

p=πm
p=π1.5
p=4.712 mm

Therefore, every pinion tooth moves the rack by:

4.712 mm

- Rack movement

The rack displacement for pinion rotation θ is:

L=θ360∘πdp

For 30 mm movement:

θ=360Lπdp
θ=36030π27
θ=127.3∘

For 50 mm movement:

θ=36050π27
θ=212.2∘

Therefore:

30 to 50 mm rack travel requires 127.3∘ to 212.2∘

- Spur gear calculation

We need a spur gear between the rack and sector.

For example, select:

Zsp=30

with:

m=1.5 mm

Pitch diameter:

dsp=mZsp
dsp=1.530
dsp=45 mm

Outside diameter:

dosp=mZsp+2
dosp=1.532
dosp=48 mm

Root diameter:

dfsp=mZsp-2.5
dfsp=1.527.5
dfsp=41.25 mm

- Torque transmitted to the spur gear

The spur gear pitch radius is:

rsp=452=22.5 mm
rsp=0.0225 m

Therefore:

Tsp=FIrsp
Tsp=297.270.0225
Tsp=6.688 N\cdotpm

With FOS:

Tsp,design=6.6882.5
Tsp,design=16.72 N\cdotpm

- Check using gear ratio

For the spur and sector:

TsTsp=ZsZsp

Therefore:

Ts=TspZsZsp
Ts=6.6886630
Ts=14.715 N\cdotpm

This exactly matches the required sector torque.

So the selected gear combination is kinematically consistent.

- Pinion torque

The pinion pitch radius is:

rp=272=13.5 mm
rp=0.0135 m

The net rack force is:

Frack=247.27 N

Therefore:

Tp=Frackrp
Tp=247.270.0135
Tp=3.338 N\cdotpm

With FOS:

Tp,design=3.3382.5
Tp,design=8.345 N\cdotpm

So the pinion should be designed for approximately:

8.35 N\cdotpm

- Tooth normal forces

Because the pressure angle is 20°:

Fn=Ftcosϕ

For the rack-spur mesh:

Fn,sp=297.27cos20∘
Fn,sp=316.35 N

With FOS:

Fn,sp,design=743.18cos20∘
Fn,sp,design=790.88 N

For the rack-pinion mesh:

Fn,p=247.27cos20∘
Fn,p=263.14 N

With FOS:

Fn,p,design=618.18cos20∘
Fn,p,design=657.86 N

- Radial forces

The radial force is:

Fr=Fttanϕ

For rack-spur:

Fr,sp=297.27tan20∘
Fr,sp=108.18 N

With FOS:

Fr,sp,design=270.45 N

For rack-pinion:

Fr,p=247.27tan20∘
Fr,p=89.98 N

With FOS:

Fr,p,design=224.95 N

- Complete force summary

| Quantity | Actual | With SF = 2.5 |
| --- | --- | --- |
| Payload force | 49.05 N | 122.63 N |
| Sector torque | 14.715 N·m | 36.788 N·m |
| Indirect rack force | 297.27 N | 743.18 N |
| Direct rack force | 50 N | 125 N |
| Net rack force | 247.27 N | 618.18 N |
| Rack-spur normal force | 316.35 N | 790.88 N |
| Rack-pinion normal force | 263.14 N | 657.86 N |
| Rack-spur radial force | 108.18 N | 270.45 N |
| Rack-pinion radial force | 89.98 N | 224.95 N |
| Spur torque | 6.688 N·m | 16.72 N·m |
| Pinion torque | 3.338 N·m | 8.345 N·m |

- Final proposed gear dimensions

| Component | Module | Teeth | Pitch diameter | Outside diameter |
| --- | --- | --- | --- | --- |
| Pinion | 1.5 mm | 18 | 27 mm | 30 mm |
| Spur gear | 1.5 mm | 30 | 45 mm | 48 mm |
| Sector gear | 1.5 mm | 66 | 99 mm | 102 mm |

The corresponding center distances are:

Pinion to rack is determined by the rack geometry.

Pinion to spur:

a=dp+dsp2
a=27+452
a=36 mm

Spur to sector:

a=45+992
a=72 mm

- The key result for your mechanism

Your 5 kg payload at a 300 mm arm produces:

14.715 N\cdotpm

At a sector pitch radius of 49.5 mm, this produces approximately:

297.3 N

downward at the rack.

Your additional 50 N direct upward force reduces the net rack force to:

297.3-50=247.3 N

With your selected safety factor:

247.3×2.5=618.2 N

However, the rack-to-spur tooth mesh still needs to withstand the full approximately 297 N transmitted force, or:

743.2 N design force

This is the critical value for sizing the rack and spur gear teeth.

⚠️ One important point: for PLA, I would not finalize the 1.5 mm module only from torque calculations. The next step should be a Lewis tooth-bending calculation for all three gear meshes, using your actual face width and 3D-printing orientation. That will tell us whether module 1.5 mm is actually strong enough for the 618 to 743 N design loads.