For this mechanism, I will use the 1.5 mm module because it gives larger and more printable teeth than module 1 mm.

The calculation below uses current design:

Rack load = 50 N upward
5 kg payload
Arm length = 300 mm
Pinion outside diameter = 30 mm
Pressure angle = 20°
Material = PLA
Factor of safety = 2.5
Required rack travel = 30 to 50 mm

The mechanism is treated as:

Rack → Spur gear → Sector gear + 300 mm arm → 5 kg payload

The pinion controls the rack motion and rotates about its own axis.

- Basic design parameters

| Parameter | Symbol | Selected value |
| --- | --- | --- |
| Module | m | 1.5 mm |
| Pressure angle | φ | 20° |
| Pinion teeth | Zₚ | 18 |
| Spur teeth | Zₛₚ | 30 |
| Sector teeth | Zₛ | 66 |
| Pinion outside diameter | dₒₚ | 30 mm |
| Spur outside diameter | dₒₛₚ | 48 mm |
| Sector outside diameter | dₒₛ | 102 mm |
| Arm length | Lₐ | 300 mm |
| Payload | M | 5 kg |
| Direct rack force | Fᴅ | 50 N |
| Safety factor | SF | 2.5 |

- Pinion tooth calculation

For a standard spur gear:

d=mZ

Outside diameter:

do=mZ+2

You specified:

do=30 mm

Therefore:

30=1.5Z+2
Z+2=20
Zp=18 teeth

Pitch diameter:

dp=mZp
dp=1.518
dp=27 mm

Pitch radius:

rp=272
rp=13.5 mm

- Pinion tooth profile

For a standard 20° full-depth involute gear:

Addendum:

ha=m
ha=1.5 mm

Dedendum:

hf=1.25m
hf=1.875 mm

Whole tooth depth:

h=2.25m
h=3.375 mm

Outside diameter:

do=30 mm

Root diameter:

df=d-2.5m
df=27-2.51.5
df=23.25 mm

Base diameter:

db=dcosϕ
db=27cos20∘
db=25.37 mm

Circular pitch:

p=πm
p=π1.5
p=4.712 mm

Tooth thickness at pitch circle:

t=p2
t=2.356 mm

- Spur gear calculation

Select:

Zsp=30

Pitch diameter:

dsp=mZsp
dsp=1.530
dsp=45 mm

Outside diameter:

do,sp=mZsp+2
do,sp=1.532
do,sp=48 mm

Root diameter:

df,sp=mZsp-2.5
df,sp=1.527.5
df,sp=41.25 mm

Base diameter:

db,sp=45cos20∘
db,sp=42.29 mm

Pitch radius:

rsp=22.5 mm

- Pinion to spur center distance

For two external gears:

a=dp+dsp2
a=27+452
a=36 mm

Therefore, the pinion and spur gear centers must be:

36 mm apart

- Sector gear calculation

The sector gear needs to generate the torque required to lift the 5 kg payload.

Payload force:

W=Mg
W=59.81
W=49.05 N

Arm length:

La=300 mm=0.3 m

Required sector torque:

Ts=WLa
Ts=49.050.3
Ts=14.715 N⋅m

- Selecting sector gear teeth

Recommended:

Zs=66

Pitch diameter:

ds=mZs
ds=1.566
ds=99 mm

Pitch radius:

rs=992
rs=49.5 mm

Outside diameter:

do,s=mZs+2
do,s=1.568
do,s=102 mm

Root diameter:

df,s=mZs-2.5
df,s=1.563.5
df,s=95.25 mm

Base diameter:

db,s=99cos20∘
db,s=93.08 mm

- Force required at sector gear

The tangential force at the sector pitch circle is:

Fs=Tsrs
Fs=14.7150.0495
Fs=297.27 N

This is the approximately 300 N force you mentioned.

Therefore:

Fs≈297 N

- Sector force with safety factor

Using:

SF=2.5
Fs,design=FsSF
Fs,design=297.272.5
Fs,design=743.18 N

The sector-spur tooth mesh should therefore be designed to withstand approximately:

743 N

- Rack force

You have two opposing forces on the rack.

Upward direct force:

FD=50 N

Downward force transmitted from the sector through the spur:

FI=297.27 N

Therefore:

FR=FI-FD
FR=297.27-50
FR=247.27 N downward

This is the net rack force.

- Rack design force with FOS

FR,design=FRSF
FR,design=247.272.5
FR,design=618.18 N

Therefore, the pinion-rack system should be designed for approximately:

618 N

net design load.

Important distinction:

The rack-spur mesh carries approximately:

297.27 N actual

or:

743.18 N design

The pinion sees the net rack force:

247.27 N actual

or:

618.18 N design

- Spur gear torque

The spur gear pitch radius is:

rsp=22.5 mm=0.0225 m

Therefore:

Tsp=Fsrsp
Tsp=297.270.0225
Tsp=6.688 N⋅m

With FOS:

Tsp,design=6.6882.5
Tsp,design=16.72 N⋅m

- Check of spur to sector gear ratio

For external gears:

TsTsp=ZsZsp

Therefore:

Ts=TspZsZsp
Ts=6.6886630
Ts=14.715 N⋅m

So the 30-tooth spur and 66-tooth sector combination produces exactly the required theoretical sector torque.

- Spur to sector center distance

asp-s=dsp+ds2
asp-s=45+992
asp-s=72 mm

- Pinion torque

The pinion experiences the net rack force:

FR=247.27 N

Pinion pitch radius:

rp=13.5 mm=0.0135 m

Therefore:

Tp=FRrp
Tp=247.270.0135
Tp=3.338 N⋅m

With FOS:

Tp,design=3.3382.5
Tp,design=8.35 N⋅m

- Rack dimensions

Rack module must match the gears:

m=1.5 mm

Rack pitch:

p=πm
p=4.712 mm

Rack tooth thickness:

t=p2
t=2.356 mm

Rack addendum:

ha=1.5 mm

Rack dedendum:

hf=1.875 mm

Total tooth depth:

h=3.375 mm

- Number of rack teeth required

For maximum required travel of 50 mm:

NR=Lp
NR=504.712
NR=10.61

You cannot use 10.61 teeth.

Use at least:

12 teeth

Rack tooth length:

LR=12p
LR=124.712
LR=56.55 mm

This provides:

56.55 mm

of theoretical tooth engagement length.

I recommend making the physical rack longer than the tooth length to provide mounting and end clearance.

A practical prototype rack can therefore be approximately:

75 to 80 mm long

with 12 teeth.

- Pinion rotation for rack movement

The fundamental relationship is:

L=θ360∘πdp

Therefore:

θ=360∘Lπdp

For 30 mm:

θ=36030π27
θ=127.3∘

For 50 mm:

θ=36050π27
θ=212.2∘

Therefore:

127.3∘≤θ≤212.2∘

- Pinion rotation direction

If the pinion rotates clockwise:

Rack moves in one direction

If the pinion reverses:

Rack moves in the opposite direction

So:

Clockwise pinion → rack upward/downward

Counterclockwise pinion → rack downward/upward

The actual direction depends on which side of the pinion the rack is positioned.

- Sector rotation

The rack moves the spur gear.

Rack displacement is related to spur rotation by:

L=rspθsp

Therefore:

θsp=Lrsp

For 50 mm:

θsp=5022.5
θsp=2.222 rad

Converting to degrees:

θsp=127.3∘

The spur gear then rotates the sector.

Gear ratio:

i=ZsZsp
i=6630
i=2.2

Therefore:

θs=θspZspZs

For 50 mm rack movement:

θs=127.33066
θs=57.86∘

So 50 mm rack travel produces approximately:

57.9∘

sector rotation.

- Tooth forces at 20° pressure angle

Normal force:

Fn=Ftcosϕ

For the rack-spur mesh:

Fn,sp=297.27cos20∘
Fn,sp=316.35 N

With FOS:

Fn,sp,design=790.88 N

Radial force:

Fr=Fttanϕ
Fr,sp=297.27tan20∘
Fr,sp=108.18 N

For the rack-pinion mesh:

Fn,p=247.27cos20∘
Fn,p=263.14 N

and:

Fr,p=247.27tan20∘
Fr,p=89.98 N

- Complete gear dimensions

| Dimension | Pinion | Spur | Sector |
| --- | --- | --- | --- |
| Module | 1.5 mm | 1.5 mm | 1.5 mm |
| Teeth | 18 | 30 | 66 |
| Pressure angle | 20° | 20° | 20° |
| Pitch diameter | 27 mm | 45 mm | 99 mm |
| Pitch radius | 13.5 mm | 22.5 mm | 49.5 mm |
| Outside diameter | 30 mm | 48 mm | 102 mm |
| Root diameter | 23.25 mm | 41.25 mm | 95.25 mm |
| Base diameter | 25.37 mm | 42.29 mm | 93.08 mm |
| Addendum | 1.5 mm | 1.5 mm | 1.5 mm |
| Dedendum | 1.875 mm | 1.875 mm | 1.875 mm |
| Total tooth depth | 3.375 mm | 3.375 mm | 3.375 mm |
| Circular pitch | 4.712 mm | 4.712 mm | 4.712 mm |
| Tooth thickness | 2.356 mm | 2.356 mm | 2.356 mm |

- Recommended face width

For your PLA prototype:

b=8 to 10 mm

I recommend:

b=10 mm

for the pinion and spur.

For the sector:

b=10 mm

This gives better load distribution and is easier to print robustly.

- Final proposed design

Pinion:

m=1.5 mm
Z=18
d=27 mm
do=30 mm
b=10 mm

Spur:

m=1.5 mm
Z=30
d=45 mm
do=48 mm
b=10 mm

Sector:

m=1.5 mm
Z=66
d=99 mm
do=102 mm
b=10 mm

Arm:

La=300 mm

Rack:

m=1.5 mm
p=4.712 mm
12 teeth minimum
Toothed length=56.55 mm
Recommended physical length≈75 to 80 mm

- Complete force and torque design values

| Parameter | Actual | Design, SF = 2.5 |
| --- | --- | --- |
| Payload force | 49.05 N | 122.63 N |
| Sector torque | 14.715 N·m | 36.79 N·m |
| Sector tangential force | 297.27 N | 743.18 N |
| Direct rack force | 50 N | 125 N |
| Net rack force | 247.27 N | 618.18 N |
| Spur torque | 6.688 N·m | 16.72 N·m |
| Pinion torque | 3.338 N·m | 8.35 N·m |
| Rack-spur normal force | 316.35 N | 790.88 N |
| Rack-pinion normal force | 263.14 N | 657.86 N |

- One important design issue

Your 300 N value is not an additional 300 N that should be added to the 50 N force.

The correct static equilibrium is:

Fnet=297.27-50=247.27 N

But the gear teeth transmitting the payload load must still be checked against approximately:

297.27 N actual

and:

743.18 N design

This distinction is important.

Also, because this is PLA and the design load is high, I would not finalize the mechanism until we perform the Lewis bending stress calculation for the pinion, spur, sector and rack. That calculation needs to use the 10 mm face width and the actual PLA allowable stress, including a suitable reduction for 3D-printing layer orientation.