Internship

Progress Report

Name: Suraj Santram Varma

Roll Number: ME4324

Designation: Student Intern

College Name: JSPM’s Rajarshi Shahu College of Engineering, Tathawade, Pune

Company Name:

Research and Development Establishment [Engineers],

Defence Research and Development Organization, Dighi, Pune – 411015

Project Statement:

Design and Development of Hydraulic Soft Actuator for Single Degree of Freedom

Content:

- Content …………………………………………………………………... 1

- Abstract ………………………………………………………………….. 2

- Introduction ……………………………………………………………… 3

- Problem Statement ………………………………………………………. 4

- Objective ………………………………………………………………… 4

- Literature Review ………………………………………………………... 5

- Derivation ………………………………………………………………... 8

- Calculations ……………………………………………………………… 13

- Pre-Testing ……………………………………………………………….. 32

- Cad Model & work-done ………………………………………………… 36

Abstract:

Soft robotic actuators have emerged as an alternative to conventional rigid actuators due to their compliance, lightweight construction, and ability to safely interact with humans. Among various soft actuators, the McKibben Artificial Muscle (MAM) has gained considerable attention because of its high force-to-weight ratio, simple construction and bio-mimetic contraction mechanism. Most existing McKibben muscles are pneumatically actuated, which limits their application in high-load systems due to the compressibility of air, lower force density, pressure fluctuations, and reduced positional accuracy.

The present research aims to design and develop a fully automatic hydraulic McKibben artificial muscle system capable of actuating a robotic elbow joint while lifting a payload of approximately 5 kg under practical operating conditions. Water or hydraulic oil is proposed as the working fluid to achieve higher force output, improved positional stability, and enhanced actuation efficiency compared to conventional pneumatic systems. In addition, the study investigates the influence of an optimized braided sheath geometry on actuator performance, including force generation, contraction ratio, stress distribution and durability.

The work involves conceptual design, CAD modeling and finite element analysis. The developed system expected to provide improved load-carrying capability, higher stiffness and better operational reliability than conventional pneumatic McKibben actuators

The proposed hydraulic artificial muscle system has potential applications in robotic manipulators. rehabilitation devices, industrial automation, wearable exoskeletons and prosthetic limbs.

Keywords: Hydraulic McKibben Artificial Muscle. Robotic Elbow Joint, Soft Robotics, Hydraulic Actuation, Braided Sheath Optimization, Finite Element Analysis

Introduction:

Soft robotic technologies have transformed the field of robotic actuation by introducing compliant mechanisms capable of interacting safely with humans and unstructured environments. Artificial muscles have become one of the most promising soft actuators because they imitate the contraction mechanism of biological skeletal muscles while maintaining low weight, compact dimensions, and high force generation.

Among different artificial muscle technologies, the McKibben Artificial Muscle is widely recognized for its simple construction and excellent force-to-weight ratio. It consists of an inner elastic bladder surrounded by a braided sleeve that converts radial expansion into axial contraction when pressurized.

Traditionally, McKibben muscles employ compressed air as the working medium. Although pneumatic systems are inexpensive and lightweight, they suffer from several disadvantages. including air compressibility, pressure losses, nonlinear response, hysteresis, limited load-carrying capability, and poor position holding under varying external loads. These limitations become increasingly significant in robotic joints that require accurate positioning and higher payload capacities.

Hydraulic actuation offers an attractive alternative because liquids are nearly incompressible. allowing higher operating pressures, improved stiffness, greater force generation, and superior positional accuracy, Consequently, hydraulic muscles have recently attracted considerable interest for rehabilitation robots, industrial manipulators, underwater robotics and wearable exoskeletons.

However, research on hydraulic muscles is limited, particularly concerning optimized braided sheath configurations, compact hydraulic systems and fully automatic. Most existing studies rely on conventional braided sleeves configurations.

This project therefore focuses on designing a fully hydraulic artificial muscle integrated with a robotic elbow mechanism capable of lifting approximately 5 kg payload. The study includes optimization of the braided sheath, hydraulic pressure analysis, finite element simulation and performance evaluation.

Problem Statement:

To design and develop a single-degree-of-freedom robotic elbow joint using a hydraulic soft actuator in an antagonistic biceps-triceps configuration, capable of lifting a minimum 5 kg load while providing the required joint torque, motion, and structural safety.

Objective:

- To study the working principle and characteristics of hydraulic McKibben muscle actuators for robotic applications.

- To design a single-DOF robotic elbow joint using a biceps-triceps type McKibben actuator arrangement for a minimum 5 kg load.

- To analyze the required joint torque, actuator force, pressure, and braided sheath parameters.

- To Finalize the suitable for mechanism for rotation of elbow.

Literature Review:

| Year | Author(s) | Research Paper / Topic | Publication | Key Finding | Limitation | Relevance |
| --- | --- | --- | --- | --- | --- | --- |
| 2022 | M. G. Antonelli et al. | Design and Characterization of a McKibben Pneumatic Muscle Prototype with an Embedded Capacitive Length Transducer | Machines, MDPI | Developed an embedded capacitive sensor capable of measuring muscle-length changes below 1 mm. | Requires a specialized embedded sensing arrangement. | Useful for future actuator displacement measurement and feedback. |
| 2018/2019 | T. Hassan et al. | Finite Element Modeling and Design of a Pneumatic Braided Muscle Actuator with Multifunctional Capabilities | IEEE/ASME Transactions on Mechatronics | Developed a bidirectional braided actuator with more than 40% stroke and demonstrated it in a 1-DOF variable-stiffness joint. | More complex sheath configuration and pneumatic operation. | Highly relevant to our 1-DOF robotic elbow and antagonistic actuator concept. |
| 2018 | M. G. Antonelli et al. | An Analytical Formula for Designing McKibben Pneumatic Muscles | International Journal of Mechanical Engineering and Technology | Developed an analytical force formula based on pressure, geometry, and contraction using FEM results. | Valid only within the investigated parameter ranges. | Useful for preliminary sizing of our actuator. |
| 2017 | M. G. Antonelli et al. | Numerical Modelling and Experimental Validation of a McKibben Pneumatic Muscle Actuator | Journal of Intelligent Material Systems and Structures | Developed nonlinear FEM considering hyperelastic tube behaviour and validated it experimentally. | Thread friction and initial tube-braid gap were not modelled. | Provides a basis for nonlinear FEM and experimental validation. |
| Year | Author(s) | Research Paper / Topic | Publication | Key Finding | Limitation | Relevance |
| 2017 | A. K. P. Ashwin, A. Ghosal | Mathematical Model for Pressure-Deformation Relationship of Miniaturized McKibben Actuators | iNaCoMM 2017 | Developed a pressure-deformation model with less than 10% standard error and predicted the locking angle near 54.7°. | Focused on miniaturized actuators and specific modelling assumptions. | Useful for understanding pressure, deformation, and braid-angle relationships. |
| 2014 | T. Nozaki, T. Noritsugu | Motion Analysis of McKibben Type Pneumatic Rubber Artificial Muscle with Finite Element Method | International Journal of Automation Technology | Developed a 3D FEM model and investigated the influence of braid angle on actuator motion. | Friction between tube, fibers, and fiber strands was not directly modelled. | Useful for braid-angle analysis and FEA methodology. |
| 2013 | I. da Paz Lopes, W. Scaff, O. Horikawa | Development A Nonconducting Manipulator, McKibben Pneumatic Artificial Muscle | COBEM 2013 | Developed and experimentally tested a nonconducting McKibben actuator for a robotic manipulator. | Relatively low force and force reduction was observed, possibly due to leakage. | Useful for actuator fabrication and robotic integration. |
| 2012 | K. Iwata, K. Suzumori, S. Wakimoto | A Method of Designing and Fabricating McKibben Muscles Driven by 7 MPa Hydraulics | International Journal of Automation Technology | Developed a hydraulic McKibben muscle and achieved about 8 kN contraction force at 7 MPa. FEM was used to optimize the sleeve angle. | Focused on high-pressure hydraulic operation. | The most directly relevant paper to our hydraulic McKibben actuator and sheath design. |
| Year | Author(s) | Research Paper / Topic | Publication | Key Finding | Limitation | Relevance |
| 2005 | R. Ramasamy et al. | An Application of Finite Element Modelling to Pneumatic Artificial Muscle | American Journal of Applied Sciences | Established FEM-based analysis for studying PAM deformation and force behaviour. | General PAM modelling with limited detailed McKibben-specific nonlinear analysis. | Provides the basic FEM foundation for our actuator analysis. |
| Table 1: Literature Survey |  |  |  |  |  |  |

Literature Survey:

- Previous studies on McKibben artificial muscles have mainly focused on understanding their working principle, mathematical modeling, and the influence of braid geometry on actuator performance. Analytical and experimental studies have established relationships between internal pressure, braid angle, contraction, actuator dimensions, and generated force. The literature also shows that the braided sheath plays a major role in converting radial expansion of the bladder into axial contraction. Finite Element Analysis has therefore been widely used to study deformation, stress distribution, and the effect of braid angle before fabrication.

- Further studies have focused on improving the design and functionality of McKibben actuators. FEM-based investigations have been used for design optimization, while analytical models have been developed for predicting force and pressure-contraction behavior. Researchers have also investigated multifunctional braided muscles capable of bidirectional motion and variable stiffness. These studies demonstrate that actuator geometry, braid configuration, and material properties strongly influence the final performance of the muscle.

- For high-force applications, hydraulic actuation has also been investigated as an alternative to conventional pneumatic McKibben muscles. The hydraulic McKibben study by Iwata et al. demonstrated operation at pressures up to 7 MPa and investigated the design and fabrication of the braided muscle using FEM-based sleeve optimization. The reviewed hydraulic soft-actuator literature further identifies high force capability as an important advantage, while also highlighting challenges such as nonlinear deformation, hysteresis, leakage, and the need for systematic design methods.

- Research Gap: Based on the reviewed literature, the present work focuses on applying a hydraulic McKibben muscle in a single-DOF robotic elbow joint using a biceps-triceps type arrangement, designed to lift a minimum 5 kg load. There is practical validation can also be done.

Derivation:

Derivation of Pressure and Force Relation:

Figure 1: McKibben PAM Components (Tondu and Lopez, 1997)

In a first approach toward the model of a McKibben type PAM, losses are not considered and thus, the gas enters the PAM and expands, generating a traction force and a work. To generate the volume increase, the gas performs a work input (Win) that can be described.

dWin = ∫(P - Pex )dli.dSi

dWin  = (P - Pex) ∫dli.dSi

dWin = P’dV

Where,

Pex = External Force,

P  = Absolute Pressure

P’ = Relative/Gauge Pressure

As the pressure performs work by volume expansion and it is assumed that the system has no loss, all the work is converted into traction and can be expressed by Wout,

dWout = -FdL

Where, F is the axial tension and dL is the displacement.

The Conservation of Energy gives:

dWin = dWout

P’dV = -FdL

F = - P’ dVdL ........................ (1)

With the strength defined in terms of the volume change, just missing define the volume variation permitted by the mesh. The variation stops when volume reaches its maximum. At this moment, the muscle reaches the largest contraction.

Figure 2: PAM Geometry (Tondu and Lopez, 1997)

Observing Fig.3, it is possible to make the length (L) and diameter (D) as a function of the angle (θ). This angle is one formed between the mesh and the axial length of the muscle. The length of fiber can be named b and n is the number of turns of the fiber around the muscle. Using b and n, it is possible to set the length and the diameter of the muscle.

cosθ= Lb  ………………………………………………………………… (1)

L = b cos….…………………………………………………………… (2)

sinθ =Dnπb

D= bsinθnπ …………….…….…………………………………………….. (3)

Making the Volume equation as function of the diameter and D & length L.

V= 14*πD2L ……………………………………………………………… (4)

By substituting D & L in eq 4

We get,

V= b34πn2 *sin2θ*cosθ…………………………………………… (5)

Diff. eq (2) & eq (5) w. r. t.

dLdθ=-bsinθ …….…..…………………………………………………… (6)

dVdθ= b34πn2 *[3cos2θ -1] …….……………………………………... (7)

Substitute the value of eq (6) & eq (7) in eq (1)

F=-P'* dVdL=-P'* dVdθdLdθ

After substituting and rearranging, we get:

F= P'D2 (3cos2θ -1)4πn2 ……………………………………………………. (8)

D= bsinθnπ

Dsinθ= bnπ

By substituting the value of D & trigonometric relation in eq (8)

F= π4 P'D2* (3cos2θ -1)sin2θ …………………………………………. (9)

Adopting contraction (ε) and applying Eq.9, it is possible to reach to required formulation (Daerden and Lefeber, 2000).

D= bsinθnπ …………………… (After applying pressure)

D0= bsinθ0nπ ………………… (Before applying pressure)

D = D0 ………………………… Basic Interpretation

So, we can replace D & sin with D0 & sin0 in eq (9)

cosθ= Lb

L = b cosθ

L0= b cosθ0

LL0= cosθcosθ0

By Rearranging,

cosθ= LL0 cosθ0 ………………………………………………………….. (10)

Where, Contraction Factor ()

ε= L0-LL0=1 - LL0

1-ε= LL0

Put this value in eq (10)

cosθ=(1-ε) * cosθ0

Substitute cos, D = Do & sin = sino in eq (9)

F= π4 P'D2* (3(1-ε)2 * cos2θ0-1)sin2θ0 ………………….......................... (11)

This Formula is for static conditions only.

By rearranging eq (11) and converting it into bar,

P'= F*4*sin2θoπ*D2*31-ϵ2*cos2θo-1*105 ………………………………. (12)

Losses:

There are various losses in calculating force, such as Braid Friction loss, Bladder elasticity loss, Hydraulic Pressure Loss, End Fitting and seal loss, and much more. But this are main losses that are considered.

Floss=Ff+Fe+Fh+Fs

Where,

Ff=Braid friction loss

Fe=Bladder elasticity loss

Fh=Hydraulic pressure loss

Fs=End fitting and sealing loss

If we want to calculate the practical force which we have to require,

Fpractical= P D024sin2θ0*31-ε2cos2θ -1-Ff-Fe-Fh-Fs

- Braid Friction loss:

Ff​=μb*Nc

Where,

μb=Braid friction coefficient

Nc=total normal contact force between the braid and bladder

- Bladder elasticity loss:

Fe​=EAε

Where,

E=Young's modulus of bladder

A=Cross-sectional area of bladder wall

ε=Axial strain

Area: A=π4Do2-Di2

Where, Doand Diare the outer and inner diameters.

- Hydraulic Pressure Loss:

Fh​=ΔPAi

Pressure drop:

ΔP=fLhDhρv22

Where,

f=Darcy friction factor

Lh=Hose length

Dh=Hose inner diameter

ρ=Fluid density

v=Fluid velocity

Ai=Internal fluid area

Internal area:

Ai=πDi24

- End Fitting and seal loss:

Fs=μs​N

Where,

μs=Seal friction coefficient

N=Normal force on seal

Let us consider, by combining all the losses in force be 30%, which simplifies our further calculation and makes easy to calculate.

Calculations:

By using this Formula:

F= π4 P'D2* (3(1-ε)2 * cos2θ0-1)sin2θ0

We can get the Force and Pressure relation.

Actuator Parameters For test trial 1:

- Length of bladder: 300mm (complete length), 250mm (active length) (we can take any length as the calculation for 0 to infinity)

L = 120mm = 0.12m

- Diameter of bladder: for trial we will go for 10mm

D = 10mm = 0.01m

- Contraction factor: ε=0.2 to 0.25

- Operating Pressure: 0 to 6 bar

- Force That required to lift the minimum of 5kg of payload

F = 5*9.81 = 49.05 N

- So, considering the required force is 10 times of payload,

F = 10*5*9.81 = 490.5 N

- Now consider gear mechanism with gear ratio 1:2, we required 0.5 times of required force,

F =0.5*490.5 =245.25 N

So, by considering Safety we will design for least 300N

After using this Formula:

F= π4 P'D2* (3(1-ε)2 * cos2θ0-1)sin2θ0

Pressure in Pascals: 1 bar = P’ * 105 Pa

We can determine the pressure value:

P'= F*4*sin2θoπ*D2*31-ϵ2*cos2θo-1*105

On D = 0.01m, o = 20 ,  = 0.2 and F = 300 N

P’ = 6.63 bar

P’ = 6.63*105 Pa

On D = 0.01m, o = 20 ,  = 0.25 and F = 300 N

P’ = 9.12 bar

P’ = 9.12*105 Pa

On D = 0.01m, o = 25 ,  = 0.2 and F = 300 N

P’ = 11.82 bar

P’ = 11.82*105Pa

On D = 0.01m, o = 25 ,  = 0.25 and F = 300 N

P’ = 17.67 bar

P’ = 17.67*105 Pa

Considering efficiency of 30% of maximum obtained value,

P’max = 17.67+0.3*17.67

P'max=22.97≈23 bar

P'max=23*105 Pa

Now, calculate for losses and what should be maximum deflection that will required for rotation, and let us verify the considered 30% loss.

First, we use Modified Formula for HSA,

Fpractical= P *105*D024sin2θ0*31-ε2cos2θ -1-(Ff-Fe-Fh-Fs)

We have P = 6.63, D0 = 10 mm = 0.01 m, θ0 = 20º, ε = 0.2

We will calculate for two different arrangements,

- Nilon 6 Braiding and Silicone Rubber Tube

- Aramid Braiding and Nitrile Butadiene Rubber (NBR) Tube

Material Properties:

| Property / Dimensions | Nilon 6 | Aramid |
| --- | --- | --- |
| Braid Friction Co-efficient (μb) | 0.25 to 0.45 | 0.14 to 0.40 |
|  | 0.35 | 0.27 |
| Diameter | 16 mm | 12.7 mm |
| Length | 250 mm | 250 mm |
| Property / Dimensions | Silicone | NBR |
| Young’s Modulus (E) | 50 MPa | 2 – 5 MPa |
|  | 50 MPa | 3.5 MPa |
| ID | 10 mm | 10 mm |
| OD | 14 mm | 12 mm |
| Length | 250 mm | 250 mm |

Table 2: Material Properties

For Arrangement 1,

Theoretical Force,

F= π4 P'D2*105*(3(1-ε)2 * cos2θ0-1)sin2θ0

F= π4 *6.63*105*0.012*(31-0.22*cos2(20)-1)sin2(20)

F=309.554 N

- Braid Friction Loss,

Ff​=μb​Nc

Ff​=0.35*100

Ff​=35 N

The value of Nc is taken as illustration; it would change as we measure the value in practical basis.

- Bladder elasticity loss:

Fe​=EAε

A=π4Do2-Di2

A=π40.0142-0.012=7.539*10-5 m2

Fe​=50*106*7.539*10-5*0.05

Fe​=188.475 N

- Hydraulic Pressure Loss:

Fh​=ΔPAi

Uses Enklo 68 Oil:

| Pipe inner diameter: | 10 mm = 0.01 m |
| --- | --- |
| Volumetric flow rate: | 0.5 L/min |
| Operating temperature: | 40°C |
| Oil density: | 890 kg/m³ |

Calculation:

V=QAi

Ai=πDi24=π*0.0124=7.854*10-5 m2

V=0.5*10-360*7.854*10-5=0.106 m/s

v=68 cSt at 40 °C

Re=VDv=0.106*0.0168*10-6=15.58

Flow is Laminar, Re < 2300

fD=64Re=6815.58=4.36

ΔP=fLhDhρv22

ΔP=4.36*0.25*890*0.10620.01*2=545 Pa

Fh​=545*7.854*10-5N

Fh​=0.0428N

- End Fitting and seal loss:

Seal Friction Co-efficient: μs​=0.4

Normal force on seal:N=50 N

Fs=μs​N

Fs=0.4*50

Fs=20 N

Now, we will calculate Practical Force value

Fpractical= F-Ff-Fe-Fh-Fs

Fpractical=309.554-35-188.475-0.0428-20

Fpractical=66.0112 N

For Arrangement 1, we got

Fpractical=66.01 N

For Arrangement 2,

Theoretical Force,

F= π4 P'D2*105*(3(1-ε)2 * cos2θ0-1)sin2θ0

F= π4 *6.63*105*0.012*(31-0.22*cos2(20)-1)sin2(20)

F=309.554 N

- Braid Friction Loss,

Ff​=μb​Nc

Ff​=0.27*100

Ff​=27 N

The value of Nc is taken as illustration, it would change as we measure the value in practical basis.

- Bladder elasticity loss:

Fe​=EAε

A=π4Do2-Di2

A=π40.0122-0.012=3.4557*10-5 m2

Fe​=3.5*106*3.4557*10-5*0.05

Fe​=6.047 N

- Hydraulic Pressure Loss:

Fh​=ΔPAi

This value will be same as we are using hydraulic fluid,

Fh​=0.0428N

- End Fitting and seal loss:

Seal Friction Co-efficient: μs​=0.4

Normal force on seal:N=50 N

Fs=μs​N

Fs=0.4*50

Fs=20 N

Now, we will calculate Practical Force value

Fpractical= F-Ff-Fe-Fh-Fs

Fpractical=309.554-35-6.047-0.0428-20

Fpractical=248.464 N

For Arrangement 2, we got

Fpractical = 248.464 N

- Rotating Mechanism calculations:

For this rotating mechanism, we will use Rack and Pinion mechanism, and one sector gear which consist arm of the hand. The pinion drives a larger spur gear, and the larger gear is attached to the arm carrying a 5 kg payload.

Design inputs:

| Payload mass | 5 kg |
| --- | --- |
| Arm length | 250 mm, assumed |
| Pitch diameters | Ø30 mm and Ø60 mm |
| Chosen Module for calculations | 2 mm |
| Pressure angle | 20° |
| Gear material | 3D-printed PLA |
| Arm and gear orientation | Centre-line angle as 50° Arm angle needs separate treatment |

Table 3: Gear Design Pre-assumptions

Min no of teeth:

zmin=2sin2φ

zmin=2sin220=17.09≈18 teeths

so, zp=18 and zg=36

Let us consider, the zp = 18 and zg = 36 (considering 1:2 teeth ratio)

The gear ratio:

i=zgzp=3618

i=2:1

By using module (m) = 2 mm and pressure angle (ϕ) = 20

Pitch diameter:

d=mz

For Pinion gear:	dp=2*18=36mm

For Spur gear:		dp=2*36=72mm

Outside diameter:

da=m(z+2)

For Pinion gear:	dp=2(18+2)=40mm

For Spur gear:		dp=2(36+2)=76mm

Approximate root diameter:

df=m(z-2.5)

For Pinion gear:	dp=2(18-2.5)=31mm

For Spur gear:		dp=2(36-2.5)=67mm

Base Diameter:

db=dcos(φ)

For Pinion gear:	dp=36*cos⁡(20)=33.83mm

For Spur gear:		dp=72*cos⁡(20)=67.66mm

Centre distance

For two external spur gears:		a=dp+dg2

a=36+722=54mm

Circular Pitch:					p=πm

p=π*2=6.283 mm

Reference tooth distance:			s=πm2

s=π*22=3.142 mm

Addendum: 			ha=m=2 mm

Dedendum:			hf=1.25*m=2.5 mm

Whole tooth depth: 		h=2.25*m=4.5 mm

Clearance: 			c=0.25*m=0.5 mm

Face width:			b=10m=10*2=20 mm

The rack geometry:

The rack and pinion must have the same module and pressure angle.

- Rack module: 2 mm

- Rack pressure angle: 20°

Calculate tooth dimensions

Rack pitch:

p=πm=6.283mm

Theoretical tooth thickness at the pitch line:

s=πm2=π*22 =3.142mm

Addendum:

ha=m=2 mm

Dedendum:

hf=1.25m=1.25(2)=2.5mm

Whole tooth depth:

h=ha+hf=2+2.5=4.5 mm

Theoretical bottom clearance:

c=0.25m=0.5 mm

The nominal tooth thickness is measured at the pitch line. Do not use 3.142 mm as the width of the tooth at every height, because the tooth profile tapers.

Rack pitch-line position relative to the pinion, for 18-tooth pinion:

rp=mzp2=2*182=18 mm

The rack pitch line must be 18 mm from the pinion center, measured perpendicular to the rack's direction of travel.

For a vertical rack, this means the horizontal distance from the pinion axis to the rack pitch line is 18 mm.

Important: The pitch line is a theoretical reference line inside the tooth geometry. It is not automatically the outside face or back face of the rack body.

Rack length and number of teeth

If the full 100 mm were toothed, the approximate number of pitches would be:

N≈1006.283=15.92 pitches

So, we can fit approximately 16 tooth pitches over 100 mm, depending on how we define the first and last tooth and the end margins.

Rack force and tooth load

Rack displacement is related to pinion rotation by:

θp=srp*180π

The larger gear rotates at half the pinion's angular displacement.

Let, s = 1mm, meaning rack moves 1 mm

θp=1*18018π=3.183°

Now, Rack moves 10 mm

θp=10*18018π=31.83°

So, we can calculate by considering the gear ratio,

| Rack movement | Pinion rotation | Spur gear rotation |
| --- | --- | --- |
| 1 mm | 3.183° | 1.592° |
| 10 mm | 31.83° | 15.92° |
| 25 mm | 79.58° | 39.79° |
| Table 4: Gear ratio and movement |  |  |

These are theoretical values without backlash or elastic deformation.

Calculate the payload torque

The payload mass is 5 kg.

F=mg

F=59.81=49.05 N

Assuming the payload centre is 180 mm from the spur gear axis and the arm is horizontal:

T=FL

T=49.05(0.18) =8.829Nm

This is the static payload torque. It does not include the weight of the arm, acceleration, impact or friction.

Apply a service factor

For preliminary sizing, assume a service factor Ks =1.5

Td=KsT

Td=1.5(8.829)=13.244 Nm

Calculate the force on the gear teeth

Tangential force is:

Ft=Tr

For the driven spur gear:

Ft=13.2440.036 =367.9N

The pinion torque for the ideal 2:1 ratio is half the spur gear torque:

Tp=13.2442=6.622Nm

The rack force is therefore:

Frack=6.6220.018 =367.9 N

The tangential load is the same at the two ideal meshes. In a real mechanism, losses and dynamic effects can change the required actuator force.

Radial force and normal tooth force

For a pressure angle of 20:

Fr=Fttan⁡(φ)

Fr=367.9tan20=133.9 N

The normal tooth contact force is:

Fn=Ftcosφ

Fn=367.9cos⁡(20)=391.6N

The forces are:

| Force | Calculated value |
| --- | --- |
| Tangential force | 367.9 N |
| Radial separating force | 133.9 N |
| Normal tooth contact force | 391.6 N |
| Table 5: Forces |  |

Select the face width:

Face width (b) is the width of the gear teeth along the gear axis.

A preliminary conventional gear guideline is:

b≈8m to 12(m)

For module 2 mm, that gives 16 to 24 mm. This is only a starting guideline. The final width must be determined from the strength calculation and the material.

Calculate tooth bending stress

Use the Lewis bending equation:

σb=FtbmY

Where:

- σb = nominal bending stress in MPa

- Ft= tangential force in N

- b = face width in mm

- m = module in mm

- Y = Lewis form factor

For preliminary calculations, use approximate 20° full-depth involute Lewis factors of 0.308 for an 18-tooth pinion, 0.376 for a 36-tooth gear, and 0.484 for a rack. Confirm the factor convention and exact tooth geometry before using these for a final design.

Pinion bending stress

σb,p=367.912*2*0.308=49.7 MPa

Spur gear bending stress

σb,g=367.912*2*0.376=40.7 MPa

Rack bending stress

σb,r=367.912*2*0.484=31.6 MPa

The pinion has the highest nominal bending stress in this comparison, so it is the first component to check.

These values are preliminary Lewis stresses. They do not include the full tooth-root stress concentration, print defects, fatigue, creep or misalignment.

Calculate the required face width

Rearrange the Lewis equation:

Ft=σmbY

For the pinion:

breq=Ftσallow*m*Y

The allowable stress must be based on testing the actual printed material and the intended loading conditions.

| Illustrative allowable stress | Required pinion width |
| --- | --- |
| 5 MPa | 119.5 mm |
| 10 MPa | 59.7 mm |
| 15 MPa | 39.8 mm |
| 20 MPa | 29.9 mm |
| Table 6: Allowable stress vs required pinion width |  |

These stress values are examples to show the calculation, not certified design limits for PLA.

A 12 mm face width produces a nominal pinion stress of about 49.7 MPa. You should not assume a typical printed PLA gear can safely withstand that stress in repeated operation.

Design implication: you may need a wider face, a larger module, lower load, a different transmission arrangement, or a more suitable material. The allowable stress must be established first.

10. Calculate load capacity and factor of safety

The nominal Lewis tangential load capacity is:

Fcap=σallowbmY

For the pinion with b=12 mm and m=2 mm. For a 3D-printed PLA gear with 20% infill, the recommended safe allowable stress is 6 to 10 MPa (870 to 1,450 psi) for standard operation, σallow=10 MPa.

Fcap=10*12*2*0308=73.2 N

That is much lower than the 367.9 N design load.

The bending factor of safety is:

FOS=σallowσb

For the pinion:

FOS=1049.7=0.2

Iterations:

| Illustrative allowable stress | Pinion FOS |
| --- | --- |
| 10 MPa | 0.20 |
| 15 MPa | 0.30 |
| 20 MPa | 0.40 |
| 30 MPa | 0.60 |
| 50 MPa | 1.01 |
| Table 7: Allowable stress vs FOS |  |

These are illustrative calculations, not verified PLA material ratings. For an uncertain printed mechanism, a preliminary target of at least 2 against a verified, condition-appropriate allowable stress is a reasonable conservative starting point.

The rack pitch line should be tangent to the pinion pitch circle, with the pitch line 18 mm from the pinion axis.

Rack displacement is related to pinion rotation by:

Rack geometry:

The rack and pinion must have the same module and pressure angle.

| Parameter | Symbol | Value |
| --- | --- | --- |
| Module | m | 2 mm |
| Pressure angle | ϕ | 20° |
| Pinion teeth | zp | 18 |
| Pinion pitch diameter | dp = mzp | 36 mm |
| Pinion pitch radius | rp=dp2 | 18 mm |
| Rack pitch | p=mπ | 6.283 mm |
| Rack tooth thickness at pitch line | s=mπ2 | 3.142 mm |
| Addendum | ha = m | 2 mm |
| Dedendum | hf = 1.25m | 2.5 mm |
| Whole tooth depth | h = 2.25 m | 4.5 mm |
| Bottom clearance | c = 0.25m | 0.5 mm |
| Table 8: Rack Parameters |  |  |

Required Rack movement to rotate the arm:

This is the equation to calculate the required rack travel for a desired arm rotation.

Since the spur gear radius is 36 mm:

s=rgθg

Here, g must be in radians.

In degrees:

s=πdgθg360

s=π*72*10360

s=6.283 mm

For your 72 mm pitch-diameter spur gear:

| Desired arm rotation | Required rack travel |
| --- | --- |
| 10° | 6.28 mm |
| 30° | 18.85 mm |
| 45° | 28.27 mm |
| 60° | 37.70 mm |
| 90° | 56.55 mm |
| Table 9: Rack rotation |  |

Braid Angle Calculation:

Initial Braid Angle:

θ0=20°

Formula to calculate Final Braid Angle:

cosθ=(1-ε) * cosθ0

Let Contraction Factor be calculated with,

ε= L0-LL0

We have,

Initial Length: L0 = 250mm

And Contracted Length: L = 200mm

ε= 250-200250=0.2

Now calculate Final Braid Angle:

cosθ=(1-ε) * cosθ0

cosθ=1-0.2*cos20=0.7517

θ=41.26°

Similarly, we can calculate from angle 5 to 54.

The things which have to keep in mind,

We want linear contraction and radial expansion.

If,

< 54.7 	(Linear Contraction and radial expansion)

> 54.7 	(Linear expansion and radial Contraction)

= 54.7	(Critical Value)

As we calculated for Initial Braid Angle 20 and Contraction Factor 0.2, there are further calculation in the table, and there is also a graphical representation of the angles and how they approach to the critical line which is 54.7 line.

Analytical Calculation:

| Initial Braid Angle (Degree) | Final Braid Angle |  |  |  |
| --- | --- | --- | --- | --- |
|  | (Contraction Factor) |  |  |  |
|  | 0.2 | 0.25 | 0.3 | 0.35 |
| 5 | 37.16 | 41.66 | 45.79 | 49.64 |
| 10 | 38.02 | 42.39 | 46.42 | 50.20 |
| 15 | 39.40 | 43.58 | 47.46 | 51.11 |
| 20 | 41.26 | 45.19 | 48.87 | 52.35 |
| 25 | 43.53 | 47.18 | 50.62 | 53.91 |
| 30 | 46.15 | 49.49 | 52.68 | 55.74 |
| 35 | 49.06 | 52.09 | 55.01 | 57.83 |
| 40 | 52.21 | 54.93 | 57.57 | 60.14 |
| 45 | 55.55 | 57.97 | 60.33 | 62.64 |
| 50 | 59.05 | 61.18 | 63.26 | 65.30 |
| 54 | 61.95 | 63.84 | 65.70 | 67.54 |
| 54.7 | 62.47 | 64.32 | 66.14 | 67.94 |
| Table 10: Angle Calculations |  |  |  |  |

Graphical Representation of Final Braid Angle vs Initial Braid Angle

Conclusion:

- Workable: 15⁰, 20⁰ and 25⁰ -> good response at ε ≤ 0.35

- Can use 20⁰ -> at ε ≤ 0.2

- < 54.7 	(Linear Contraction and radial expansion)

- > 54.7 	(Linear expansion and radial Contraction)

- = 54.7	(Critical Value)

Different Iterations on diameter variation:

Iteration 1:

| Initial Braid Angle (0) (Degree) | Pressure in bar |  |  |  | Pressure Value (Logarithmic) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (Contraction Ratio) |  |  |  | (Contraction Ratio) |  |  |  |
|  | 0.2 | 0.25 | 0.3 | 0.35 | 0.2 | 0.25 | 0.3 | 0.35 |
| 5 | 0.320 | 0.430 | 0.632 | 1.125 | -0.494 | -0.366 | -0.199 | 0.051 |
| 10 | 1.336 | 1.809 | 2.706 | 5.023 | 0.126 | 0.257 | 0.432 | 0.701 |
| 15 | 3.233 | 4.454 | 6.887 | 14.013 | 0.510 | 0.649 | 0.838 | 1.147 |
| 20 | 6.425 | 9.117 | 14.992 | 37.475 | 0.808 | 0.960 | 1.176 | 1.574 |
| 25 | 11.822 | 17.670 | 32.886 | 165.924 | 1.073 | 1.247 | 1.517 | 2.220 |
| 30 | 21.703 | 35.950 | 93.164 | -193.403 | 1.337 | 1.556 | 1.969 | 2.286 |
| 35 | 43.582 | 94.964 | -922.975 | -84.060 | 1.639 | 1.978 | 2.965 | 1.925 |
| 40 | 124.561 | -1621.285 | -114.889 | -61.601 | 2.095 | 3.210 | 2.060 | 1.790 |
| 45 | -477.465 | -122.231 | -72.070 | -52.146 | 2.679 | 2.087 | 1.858 | 1.717 |
| 50 | -108.441 | -74.034 | -57.089 | -47.061 | 2.035 | 1.869 | 1.757 | 1.673 |
| 54 | -74.261 | -59.955 | -50.801 | -44.478 | 1.871 | 1.778 | 1.706 | 1.648 |
| 54.7 | -70.895 | -58.286 | -49.971 | -44.113 | 1.851 | 1.766 | 1.699 | 1.645 |
| Table 11: Initial Braid Angle vs Pressure |  |  |  |  |  |  |  |  |

Conclusion:

- Pressure becomes negative when the braid angle is 54.7° or more.

- The actuator works safely below 54.7°.

- A 25° initial braid angle is selected because it remains safe up to 35% contraction strain.

- The actuator shows linear contraction below 54.7° and tends to expand above 54.7°.

- Theoretical analysis was also carried out for actuator diameters of 16, 20, 25, 30, 40, 50, and 60 mm.

- By this way we can determine with multiple iterations.

Graphical Representation of Initial Braid Angle vs Logarithmic Value of Pressure on contraction factor 0.2 or 20%

Graphical Representation of Initial Braid Angle vs Logarithmic Value of Pressure on contraction factor 0.25 or 25%

Graphical Representation of Initial Braid Angle vs Logarithmic Value of Pressure on contraction factor 0.3 or 30%

Graphical Representation of Initial Braid Angle vs Logarithmic Value of Pressure on contraction factor 0.35 or 35%

Theoretical & Practical Pre-testing:

Formula

ΔLth=ε×L0

Given Data

- Active length, L0=120 mm

- Contracted Length, L = 96 mm

- Contraction strain, ε=20%

Calculation

ΔLth=0.2×120=24 mm

Practical Test Results:

| Pressure (kg/cm2) | NBR-Aramid (mm) | Silicone-Nylon (mm) |
| --- | --- | --- |
| 2 | 0 | 2 |
| 3 | 0 | 5 |
| 4.5 | — | 15 |
| 5 | 10 | 20 |
| 6 | 25 | — |
| 10 | 30 | 20 |
| Table 12: Test results |  |  |

1 kg/cm2 = 0.98 bar

In practical pre-testing we have used two different rubber tubes and two different braiding. And paired with each other to obtain deflection under the zero load.

Rubber Tubes:

- Silicon Rubber Tube (OD = 14mm, ID = 10mm)

- Nitrile Butadiene Rubber (NBR) Tube (OD = 12mm, ID = 10mm)

Braiding Material:

- Nilon 6 (ID = 16mm)

- Aramid (ID = 12.7mm)

Configuration and arrangement used:

- Silicon rubber tube with Nilon 6 Braiding

- NBR tube with Aramid Braiding

Fluid that used: HP Enklo 68

Before Contraction			After Contraction

Figure 3: Image Before and After Contraction

Graphical Representation (Pressure vs Stroke):

Graphical Representation (Pressure vs Contracted Length):

Maximum Length Change Comparison:

| Type | Length Change (mm) |
| --- | --- |
| Theoretical | 24 |
| NBR-Aramid | 30 |
| Silicone-Nylon | 20 |
| Table 13: Length Comparison |  |

Images of Pre-testing:

Figure 4: Pre-Test set-up for Silicon Tube and Nylon Braiding

Figure 5: Pre-Test set-up for NBR Tube and Aramid Braiding

CAD Models:

Figure 6: HSA Sheath

Figure 7: HSA Bladder

Figure 8: HSA Sheath and Bladder Mechanism

Rotating Mechanism for Arm:

We are planning to use rack and pinion mechanism, which a sector gear being a Forearm of the hand.

Further are some images of the gear mechanism which had done till now.

Figure 9: Reference Dimensions

Figure 10: Sector Gear, Rack and Pinion arrangement

Figure 11: Image of guide and connector for gears and rack

Figure 12: Complete arm mechanism

- This Kind of assembly we have to make:

Like mostly of Biceps and triceps, using hydraulic (Water/Oil) as a working fluid for actuate the muscle.

- Hence forth the mechanism and hydraulic power pack we have to design requiring for this actuator.

- And enabling it’s SDOF lifting.

Figure 13: Reference from google