PHASE 1: Literature Study, Theoretical Analysis and Preliminary Design

Duration: July to August

Status: Completed

Objective: To establish the theoretical and design basis of the hydraulic McKibben muscle actuator.

Work Completed

- Literature review of McKibben muscle actuators.

- Study of pneumatic and hydraulic actuation.

- Study of the effect of pressure on actuator force.

- Study of braid geometry and braid angle.

- Study of actuator contraction.

- Theoretical force calculations.

- Pressure and contraction calculations.

- Preliminary actuator sizing.

- CAD modelling of the bladder & CAD modelling of the braided sheath.xxxxxx

- Preliminary actuator assembly development.

Preliminary Actuator Design Parameters

| Parameter | Value |
| --- | --- |
| Actuator type | Hydraulic McKibben muscle |
| Total actuator length | 300 mm |
| Active actuator length | 250 mm |
| Upper end section | 25 mm |
| Lower end section | 25 mm |
| Nominal initial diameter | 40 mm |
| Target contraction | 20 to 25% |
| Maximum contraction displacement | 50 to 62.5 mm |
| Minimum operating pressure | 0 bar |
| Maximum operating pressure | 6 bar |
| Pressure increment | 1.5 bar |
| Test pressure levels | 0, 1.5, 3, 4.5, 6 bar |
| Initial test load | 0 kg |
| Minimum payload considered | 5 kg |
| Equivalent payload force | 49.05 N |
| Test orientation | Vertical |

Dimensional Relationship

Ltotal = Lactive + Le1 + Le2

300 mm = 250 mm + 25 mm + 25 mm

PHASE 2: Final Actuator Specification and Specimen Preparation

Duration: September, Week 1

Objective: To freeze the actuator geometry and prepare the test specimen before experimental testing.

Activities

- Finalize bladder material.

- Finalize bladder wall thickness.

- Measure bladder inner and outer diameter.

- Finalize braided sheath material.

- Measure braid length.

- Measure initial braid angle.

- Finalize end fitting geometry.

- Assemble the complete actuator.

- Assign a specimen identification number.

- Record all initial dimensions.

Specimen Specification Table

| Component | Parameter | Value |
| --- | --- | --- |
| Bladder | Material | Final selected material |
| Bladder | Total length | 300 mm |
| Bladder | Active length | 250 mm |
| Bladder | Outer diameter | 40 mm nominal |
| Bladder | Inner diameter | Measured after material selection |
| Braided sheath | Material | Final selected material |
| Braided sheath | Active length | 250 mm |
| Braided sheath | Initial braid angle | 20 & 25 |
| Braided sheath | Initial diameter | Measured after assembly |
| Upper end fitting | Length | 25 mm |
| Lower end fitting | Length | 25 mm |
| Complete actuator | Total length | 300 mm |

Output

Complete actuator specimen specification sheet.

PHASE 3: Test Rig Fabrication and Commissioning

Duration: September, Week 2

Objective: To develop and validate a vertical test rig for measuring pressure, force, contraction, deformation, and structural response.

Test Rig Configuration

The rig should include:

- Fixed upper mounting.

- Lower moving mounting.

- Vertical guide arrangement.

- Load application arrangement.

- Pressure supply.

- Pressure measurement.

- Force measurement.

- Displacement measurement.

- Diameter measurement procedure.

- Safety enclosure.

Test Rig Requirements

| Parameter | Requirement |
| --- | --- |
| Maximum actuator length | 300 mm |
| Maximum expected contraction | 62.5 mm |
| Recommended displacement range | 0 to 100 mm minimum |
| Maximum operating pressure | 6 bar |
| Recommended pressure sensor range | Minimum 0 to 10 bar |
| Minimum payload | 5 kg |
| Equivalent load | 49.05 N |
| Recommended force sensor capacity | Minimum 100 N |
| Test orientation | Vertical |
| Upper mounting | Fixed |
| Lower mounting | Moving |
| Alignment | Concentric with actuator axis |

Activities

- Fabricate the frame.

- Install upper and lower fixtures.

- Check concentricity.

- Install pressure measurement.

- Install force measurement.

- Install displacement measurement.

- Check load application.

- Conduct dry-run testing.

- Calibrate instruments.

Output

Commissioned test rig.

PHASE 4: Baseline Characterization and Leakage Testing

Duration: September, Week 3 to Week 4

Objective: To establish the initial physical condition of the actuator before integrity testing.

Initial Measurements

| Parameter | Measurement |
| --- | --- |
| Total length | mm |
| Active length | mm |
| Initial diameter | mm |
| Bladder wall thickness | mm |
| Initial braid angle | Degree |
| Actuator mass | kg or g |
| Initial external load | 0 kg |

Test 1: Visual Inspection

Inspect:

- Bladder surface.

- Braided sheath.

- End fittings.

- Sealing regions.

- Braid uniformity.

Test 2: Leakage Test

Apply pressure incrementally.

Leakage Test Matrix

| Test ID | Pressure | External Load | Hold Condition | Observation |
| --- | --- | --- | --- | --- |
| B1 | 0 bar | 0 kg | Initial | Baseline |
| B2 | 1.5 bar | 0 kg | Hold | Leakage check |
| B3 | 3 bar | 0 kg | Hold | Leakage check |
| B4 | 4.5 bar | 0 kg | Hold | Leakage check |
| B5 | 6 bar | 0 kg | Hold | Leakage check |

Measurements

Pressure, leakage, actuator length, diameter, and visible deformation.

Output

Baseline condition and initial pressure response.

PHASE 5: Elastic Integrity Validation

Duration: October, Week 1 to Week 3

Objective: To determine the elastic behaviour and recovery characteristics of the actuator under repeated pressurization.

Constant Parameters

| Parameter | Value |
| --- | --- |
| Total actuator length | 300 mm |
| Active actuator length | 250 mm |
| Initial nominal diameter | 40 mm |
| Test orientation | Vertical |
| Maximum operating pressure | 6 bar |
| Pressure levels | 0, 1.5, 3, 4.5, 6 bar |
| Initial external load | 0 kg |

Test 1: Pressure Versus Contraction

| Test ID | Pressure | Initial Length | Contraction |
| --- | --- | --- | --- |
| E1 | 0 bar | L0 | 0% |
| E2 | 1.5 bar | L0 | Calculate |
| E3 | 3 bar | L0 | Calculate |
| E4 | 4.5 bar | L0 | Calculate |
| E5 | 6 bar | L0 | Calculate |

Contraction:

ε = (L0 − L) / L0 × 100

Test 2: Radial Expansion

| Test ID | Pressure | Initial Diameter | Pressurized Diameter | Radial Change |
| --- | --- | --- | --- | --- |
| R1 | 0 bar | D0 | D | 0 |
| R2 | 1.5 bar | D0 | D | Calculate |
| R3 | 3 bar | D0 | D | Calculate |
| R4 | 4.5 bar | D0 | D | Calculate |
| R5 | 6 bar | D0 | D | Calculate |

Test 3: Loading and Unloading

Apply: 0 → 1.5 → 3 → 4.5 → 6 bar

Then: 6 → 4.5 → 3 → 1.5 → 0 bar

Measure contraction at every pressure level.

Test 4: Elastic Recovery

After complete depressurization, measure:

Final recovered length, Lr

Final recovered diameter, Dr

Calculate residual deformation.

Test 5: Repeatability

| Test ID | Pressure | Number of Cycles |
| --- | --- | --- |
| ER1 | 3 bar | 5 |
| ER2 | 3 bar | 10 |
| ER3 | 4.5 bar | 25 |
| ER4 | 4.5 bar | 50 |

Output

- Pressure versus contraction curve.

- Pressure versus diameter curve.

- Hysteresis curve.

- Elastic recovery data.

- Residual deformation data.

- Repeatability data.

PHASE 6: Structural Integrity Validation

Duration: October, Week 4 to November, Week 2

Objective: To validate the physical and structural stability of the complete actuator.

Constant Parameters

| Parameter | Value |
| --- | --- |
| Total actuator length | 300 mm |
| Active actuator length | 250 mm |
| Nominal diameter | 40 mm |
| Maximum operating pressure | 6 bar |
| Test orientation | Vertical |
| Actuator alignment | Concentric |
| Inspection | Before and after each test |

Test 1: Pressure Ramp Test

Increase pressure from 0 to 6 bar.

Pressure levels: 0, 1.5, 3, 4.5, 6 bar.

Observe: Leakage, Local bulging, Braid movement, Braid slippage, End-fitting movement, Structural deformation

Test 2: Pressure Hold Test

| Test ID | Pressure | External Load | Measurement |
| --- | --- | --- | --- |
| S1 | 3 bar | 0 kg | Pressure stability |
| S2 | 4.5 bar | 0 kg | Pressure stability |
| S3 | 6 bar | 0 kg | Pressure stability |

Record pressure drop, length change, diameter change, and leakage.

Test 3: External Load Test

Use loads up to the design payload.

| Test ID | External Load | Equivalent Force |
| --- | --- | --- |
| L1 | 0 kg | 0 N |
| L2 | 1 kg | 9.81 N |
| L3 | 2 kg | 19.62 N |
| L4 | 3 kg | 29.43 N |
| L5 | 4 kg | 39.24 N |
| L6 | 5 kg | 49.05 N |

Test 4: Combined Pressure and Load Test

Test each load at selected pressure levels.

| Pressure | Load Range |
| --- | --- |
| 1.5 bar | 0 to 5 kg |
| 3 bar | 0 to 5 kg |
| 4.5 bar | 0 to 5 kg |
| 6 bar | 0 to 5 kg |

Measure: Force, Contraction, Diameter, Leakage, Structural deformation

Output

- Structural integrity assessment.

- Safe pressure range.

- Safe load range.

- Observed structural failure modes.

PHASE 7: Force and Contraction Performance Testing

Duration: November, Week 3 to Week 4

Objective: To experimentally validate force generation and contraction.

Test Matrix

| Pressure | 0 kg | 1 kg | 2 kg | 3 kg | 4 kg | 5 kg |
| --- | --- | --- | --- | --- | --- | --- |
| 0 bar | Test | Test | Test | Test | Test | Test |
| 1.5 bar | Test | Test | Test | Test | Test | Test |
| 3 bar | Test | Test | Test | Test | Test | Test |
| 4.5 bar | Test | Test | Test | Test | Test | Test |
| 6 bar | Test | Test | Test | Test | Test | Test |

Tests: Free contraction test, Loaded contraction test, Force versus pressure test, Contraction versus pressure test, Blocked-force test, if the rig and sensor arrangement support it.

Output

- Force versus pressure graph.

- Contraction versus pressure graph.

- Load versus contraction graph.

- Experimental versus theoretical comparison.

PHASE 8: Cyclic and Endurance Testing

Duration: December, Week 1 to Week 2

Objective: To determine degradation of actuator performance under repeated operation.

Cyclic Test Matrix

| Test ID | Pressure | External Load | Number of Cycles |
| --- | --- | --- | --- |
| C1 | 3 bar | 0 kg | 100 |
| C2 | 3 bar | Selected load | 500 |
| C3 | 4.5 bar | Selected load | 1,000 |
| C4 | 6 bar | Selected safe load | Final endurance cycle block |

After each cycle block, measure: Maximum contraction, Generated force, Recovered length, Recovered diameter, Leakage, Braid condition, End-fitting condition

Output

- Cycle versus contraction.

- Cycle versus force.

- Performance degradation.

- Elastic degradation.

- Structural degradation.

- Failure observations.

PHASE 9: Final Validation and Post-Test Inspection

Duration: December, Week 3

Objective :To compare the actuator condition before and after all testing.

Final Inspection Matrix

| Parameter | Before Testing | After Testing |
| --- | --- | --- |
| Total length | Ltotal | Lfinal |
| Active length | Lo | Lfinal |
| Diameter | Do | Dfinal |
| Wall thickness | To | tfinal |
| Braid angle | θo | θfinal |
| Maximum contraction | Initial | Final |
| Maximum force | Initial | Final |
| Leakage condition | Initial | Final |
| Bladder condition | Initial | Final |
| Braid condition | Initial | Final |
| End fitting condition | Initial | Final |

Final Tests

- Leakage test.

- Pressure versus contraction test.

- Pressure versus force test.

- Elastic recovery test.

- Short repeatability test.

- Final structural inspection.

Output

- Final actuator condition.

- Validated operating range.

- Performance retention.

- Elastic integrity assessment.

- Structural integrity assessment.

PHASE 10: Data Analysis and Final Documentation

Duration: December, Week 4

Objective: To analyze the complete experimental programme.

Activities

- Plot pressure versus contraction.

- Plot pressure versus force.

- Plot pressure versus diameter.

- Calculate hysteresis.

- Calculate elastic recovery.

- Calculate residual deformation.

- Compare pre-cycle and post-cycle performance.

- Compare theoretical and experimental force.

- Identify structural damage.

- Identify failure modes.

- Define the experimentally validated operating range.

Final Deliverables

- Complete test matrix.

- Test rig specification.

- Specimen dimensional drawing.

- Pressure-contraction curves.

- Pressure-force curves.

- Pressure-diameter curves.

- Hysteresis curves.

- Cyclic degradation curves.

- Elastic integrity report.

- Structural integrity report.

- Final validated McKibben muscle specification.