Phase 1: Literature Study, Theoretical Analysis and Initial Design

July to August, Completed

Objective: To understand the working principle and characteristics of hydraulic McKibben muscle actuators and establish the theoretical basis for actuator design.

Activities

- Studied the McKibben muscle working principle.

- Conducted literature review on pneumatic and hydraulic McKibben muscles.

- Studied the influence of internal pressure, braid angle, diameter, length, and contraction.

- Studied the influence of braided sheath geometry on actuator performance.

- Derived the pressure-force relationship, and also derived the geometrical relationship between braid angle, diameter, and actuator length.

- Studied the contraction relationship.

- Performed preliminary actuator sizing calculations.

- Prepared CAD models of the bladder, braided sheath.

- Prepared the preliminary McKibben muscle assembly.

Preliminary Actuator Parameters

| Parameter | Value |
| --- | --- |
| Total bladder length | 300 mm |
| Active actuator length | 250 mm |
| Trial bladder diameter | 40 mm |
| Initial contraction range | 20% to 25% |
| Preliminary operating pressure | 0 to 6 bars |
| Required payload | Minimum 5 kg |
| Initial Braid Angle | 20 to 25 |
| Preliminary required force considered | 490.5 N |

Output

- Literature review

- Theoretical derivation

- Force-pressure relationship & Contraction relationship

- Preliminary actuator dimensions

- CAD model of bladder

- CAD model of braided sheath

- Preliminary actuator assembly

Phase 2: Test-Rig Design and Commissioning

September, Week 1 to Week 2

Objective: To develop and validate a safe test setup for measuring actuator force, contraction, radial expansion, pressure response, and structural behavior.

Test Setup

The setup will include:

- Vertical actuator mounting structure

- Fixed upper mounting

- Lower moving attachment

- Load application arrangement

- Pressure supply system

- Pressure gauge or pressure sensor

- Pressure control arrangement

- Force measurement arrangement

- Linear displacement or contraction measurement

- Diameter measurement arrangement

- Safety enclosure

Test Setup Parameters

| Parameter | Requirement |
| --- | --- |
| Actuator mounting | Vertical |
| Actuator alignment | Concentric and aligned with load axis |
| Pressure measurement | Within selected operating range |
| Force measurement | Suitable for expected actuator force |
| Displacement measurement | Suitable for expected contraction |
| Load application | Incremental dead weights or equivalent |
| Safety provision | Protective enclosure or shielding |
| Test specimen | Preliminary hydraulic McKibben actuator |
| Total actuator length | 300 mm |
| Active actuator length | 250 mm |
| Nominal trial diameter | 40 mm |
| Braid configuration | Cross Linking configuration |
| Bladder material | PU, TPU & Silicon |
| Working fluid | Water or hydraulic fluid |
| Test orientation | Vertical |
| Initial braid angle | 20 & 25 |
| Initial external load | 0 to 5kg |

Activities

- Finalize vertical test rig design.

- Fabricate or assemble the test structure.

- Ensure actuator alignment with the loading axis.

- Install the pressure supply system.

- Install pressure measurement.

- Install the load application arrangement.

- Install displacement measurement.

- Prepare diameter measurement procedure.

- Prepare data recording format.

- Perform dry-run testing.

Output

- Validated test rig

- Test setup drawing

- Instrument arrangement

- Standard test procedure

- Initial test matrix

Phase 3: Baseline Characterization and Initial Pressure Testing

September, Week 3 to Week 4

Objective: To establish the initial physical condition and pressure response of the actuator before detailed integrity testing.

Constant Parameters

| Parameter | Requirement |
| --- | --- |
| Total actuator length | 300 mm |
| Active actuator length | 250 mm |
| Initial braid angle | 20 & 25 |
| Contraction Factor | 0.2 to 0.25 |
| Nominal trial diameter | 40 mm |
| Braid configuration | Cross Linking configuration |
| Bladder material | PU & Silicon |
| Working fluid | Water or hydraulic fluid |
| Test orientation | Vertical |
| Initial external load | 0 to 5kg |

Initial Dimensional Inspection

| Parameter | Measurement |
| --- | --- |
| Total length | L₀ |
| Active length | Lₐ |
| Outer diameter | D₀ |
| Bladder wall thickness | t |
| Initial braid angle | θ₀ |
| Braid condition | Visual inspection |
| End fitting condition | Visual inspection |

Tests

- Dimensional inspection.

- Visual inspection.

- Initial leakage test.

- Low-pressure actuation.

- Pressure versus contraction measurement.

- Pressure versus diameter measurement.

- End-fitting stability inspection.

Pressure Test Matrix

| Test ID | Pressure Level | External Load | Measurement |
| --- | --- | --- | --- |
| B1 | 0 bar | No load | Initial dimensions |
| B2 | 1.5 bar | No load | Length and diameter |
| B3 | 3.0 bar | No load | Length and diameter |
| B4 | 4.5 bar | No load | Length and diameter |
| B5 | 6.0 bar | No load | Length and diameter |
| B6 | 7.5 bar | No load | Length and diameter |
| B7 | 9.0 bar | No load | Length and diameter |

Test 2: Leakage Test

Apply pressure incrementally.

Leakage Test Matrix

| Test ID | Pressure | External Load | Hold Condition | Observation |
| --- | --- | --- | --- | --- |
| B1 | 0 bar | 0 kg | Initial | Baseline |
| B2 | 1.5 bar | 0 kg | Hold | Leakage check |
| B3 | 3 bar | 0 kg | Hold | Leakage check |
| B4 | 4.5 bar | 0 kg | Hold | Leakage check |
| B5 | 6 bar | 0 kg | Hold | Leakage check |

Measurements

- Pressure

- Actuator length

- Contraction

- Diameter

- Leakage

- Visible deformation

Output

- Baseline actuator specification

- Initial pressure-contraction curve

- Pressure-diameter curve

- Initial leakage condition

Phase 4: Elastic Integrity Validation

October, Week 1 to Week 3

Objective: To determine the elastic deformation, recovery, hysteresis, repeatability, and residual deformation characteristics of the McKibben muscle.

Constant Parameters

| Parameter | Requirement |
| --- | --- |
| Total actuator length | 300 mm |
| Active length | 250 mm |
| Nominal diameter | 40 mm |
| Actuator configuration | Same selected configuration |
| Working fluid | Same selected fluid |
| Test orientation | Vertical |
| External load | No load for primary elastic test |

Test 1: Pressure-Deformation Test

Apply pressure in controlled increments and measure the actuator response.

Test 1: Pressure Versus Contraction

| Test ID | Pressure | Initial Length | Contraction |
| --- | --- | --- | --- |
| E1 | 0 bar | L0 | 0% |
| E2 | 1.5 bar | L0 | Calculate |
| E3 | 3 bar | L0 | Calculate |
| E4 | 4.5 bar | L0 | Calculate |
| E5 | 6 bar | L0 | Calculate |

Contraction:

ε = (L0 − L) / L0 × 100

Test 2: Radial Expansion

| Test ID | Pressure | Initial Diameter | Pressurized Diameter | Radial Change |
| --- | --- | --- | --- | --- |
| R1 | 0 bar | D0 | D | 0 |
| R2 | 1.5 bar | D0 | D | Calculate |
| R3 | 3 bar | D0 | D | Calculate |
| R4 | 4.5 bar | D0 | D | Calculate |
| R5 | 6 bar | D0 | D | Calculate |

Test 3: Loading and Unloading Test

Apply pressure incrementally up to the selected maximum operating pressure.

Then reduce the pressure through the same levels.

Apply: 0 → 1.5 → 3 → 4.5 → 6 bar

Then: 6 → 4.5 → 3 → 1.5 → 0 bar

Measure:

- Pressure

- Contraction

- Diameter

- Recovery

Test 4: Elastic Recovery Test

After complete depressurization, measure:

- Final length

- Final diameter

- Residual axial deformation

- Residual radial deformation

Test 5: Repeatability Test

Repeat selected loading and unloading cycles.

Suggested initial test sequence:

| Test | Number of Cycles |
| --- | --- |
| R1 | 5 |
| R2 | 10 |
| R3 | 25 |
| R4 | 50 |

Measurements

Maximum contraction, Maximum diameter, Final recovered length, Final recovered diameter, Residual deformation, Variation between cycles

Output

- Pressure versus contraction curve

- Pressure versus diameter curve

- Loading and unloading curve

- Hysteresis curve

- Elastic recovery results

- Residual deformation results

- Repeatability data

Phase 5: Structural Integrity Validation

October, Week 4 to November, Week 2

Objective: To determine the structural stability and safe operating condition of the complete hydraulic McKibben muscle.

Constant Parameters

| Parameter | Requirement |
| --- | --- |
| Total actuator length | 300 mm |
| Active length | 250 mm |
| Nominal diameter | 40 mm |
| Actuator configuration | Selected configuration |
| Test orientation | Vertical |
| Maximum operating pressure | 6 bar |
| Actuator alignment | Concentric |
| Inspection | Before and after each test |

Test 1: Pressure Ramp Test

Increase pressure gradually.

Increase pressure from 0 to 6 bar.

Pressure levels: 0, 1.5, 3, 4.5, 6 bar.

Observe:

- Bladder expansion

- Local bulging

- Braid movement

- Braid slippage

- Leakage

- End-fitting movement

Test 2: Pressure Holding Test

Maintain selected pressure for a predetermined duration, atleast 5-10 secs.

Pressure Hold Matrix

| Test ID | Pressure Level | Hold Condition | Measurements |
| --- | --- | --- | --- |
| S1 | 3.0 bar | Selected duration | Pressure decay, deformation |
| S2 | 4.5 bar | Selected duration | Pressure decay, deformation |
| S3 | 6.0 bar | Selected duration | Pressure decay, deformation |

Test 3: External Load Test

Apply incremental external loads to the unpressurized and pressurized actuator where applicable, from 0 to 5kg load.

Load Matrix

| Test ID | External Load | Equivalent Force |
| --- | --- | --- |
| L1 | 0 kg | 0 N |
| L2 | 1 kg | 9.81 N |
| L3 | 2 kg | 19.62 N |
| L4 | 3 kg | 29.43 N |
| L5 | 4 kg | 39.24 N |
| L6 | 5 kg | 49.05 N |

Test 4: Combined Pressure and Load Test

Apply pressure while the actuator is subjected to an external load.

Test each load at selected pressure levels.

| Pressure | Load Range |
| --- | --- |
| 1.5 bar | 0 to 5 kg |
| 3 bar | 0 to 5 kg |
| 4.5 bar | 0 to 5 kg |
| 6 bar | 0 to 5 kg |

Measure: Pressure, Force, Contraction, Diameter, Leakage, Structural deformation

Post-Test Inspection

Inspect:

- Bladder

- Braided sheath

- End fittings

- Braid slippage

- Braid fraying

- Local bulging

- Leakage points

- Permanent deformation

Output

- Structural integrity assessment

- Safe operating condition

- Pressure and load limits

- Structural failure observations

- Post-test dimensional comparison

Phase 6: Force-Contraction Characterization

November, Week 3 to Week 4

Objective: To experimentally determine the relationship between pressure, force, external load, and contraction.

Constant Parameters

| Parameter | Requirement |
| --- | --- |
| Total actuator length | 300 mm |
| Active length | 250 mm |
| Nominal diameter | 40 mm |
| Actuator configuration | Selected configuration |
| Test orientation | Vertical |
| Pressure range | Selected validated range |

Test Conditions

External Load Matrix

| Pressure Level | No Load | 1 kg | 2 kg | 3 kg | 4 kg |
| --- | --- | --- | --- | --- | --- |
| 0 bar | Test | Test | Test | Test | Test |
| 1.5 bar | Test | Test | Test | Test | Test |
| 3.0 bar | Test | Test | Test | Test | Test |
| 4.5 bar | Test | Test | Test | Test | Test |
| 6.0 bar | Test | Test | Test | Test | Test |

Tests: Free contraction test, Loaded contraction test, Force versus pressure test, Contraction versus pressure test, Blocked-force test, if the rig and sensor arrangement support it.

Measurements: Pressure, Generated force, External load, Actuator length, Contraction, Diameter

Output

- Force versus pressure curve

- Contraction versus pressure curve

- Force versus contraction curve

- Load versus contraction curve

- Experimental and theoretical comparison

Phase 7: Cyclic and Endurance Testing

December, Week 1 to Week 2

Objective: To evaluate the degradation of elastic and structural performance under repeated actuator operation.

Initial Conditions

| Parameter | Requirement |
| --- | --- |
| Actuator | Selected validated configuration |
| Pressure | Selected operating pressure |
| External load | Selected load condition |
| Test orientation | Vertical |
| Measurements | Before, during, and after cycle blocks |

Test Matrix

| Test ID | Pressure Level | External Load | Number of Cycles | Purpose |
| --- | --- | --- | --- | --- |
| C1 | 3 bar | 0 kg | 100 | Initial cyclic behaviour |
| C2 | 3 bar | Selected load | 500 | Intermediate endurance |
| C3 | 4.5 bar | Selected load | 1,000 | Extended cyclic behaviour |
| C4 | 6 bar | Selected safe load | Further cycles if time permits | Degradation evaluation |

Parameters to Monitor: Maximum contraction, Generated force, Recovered length, Recovered diameter, Leakage, Braid condition, End-fitting condition, Hysteresis change

Inspection

Conduct inspection:

- Before testing

- After each cycle block

- After final cyclic testing

Output

- Cycle versus contraction curve

- Cycle versus force curve

- Performance degradation data

- Elastic degradation data

- Structural degradation data

- Failure mode observations

Phase 8: Final Validation and Proof Testing

December, Week 3

Objective: To validate the final actuator configuration after elastic, structural, and cyclic testing.

Final Actuator Parameters

| Parameter | Final Value |
| --- | --- |
| Total length | Final measured value |
| Active length | Final measured value |
| Outer diameter | Final measured value |
| Wall thickness | Final measured value |
| Braid angle | Final measured value |
| Operating pressure range | Experimentally validated range |
| Maximum validated force | Experimental result |
| Maximum contraction | Experimental result |

Tests

- Final dimensional inspection.

- Leakage test.

- Pressure-contraction test.

- Pressure-force test.

- Elastic recovery test.

- Short repeatability test.

- Structural inspection.

- Controlled proof-pressure test according to the finalized test procedure.

Proof Test Procedure

- Start from zero pressure.

- Increase pressure in controlled increments.

- Hold at predefined pressure levels.

- Inspect the actuator.

- Record deformation and leakage.

- Stop according to predefined safety or failure criteria.

Output

- Final validated actuator configuration

- Final operating pressure range

- Final force capability

- Final contraction capability

- Elastic integrity assessment

Phase 9: Data Analysis and Final Documentation

December, Week 4

Objective: To analyze the complete experimental data and establish the final performance and integrity characteristics of the hydraulic McKibben muscle.

Analysis

- Compare theoretical and experimental force.

- Compare theoretical and experimental contraction.

- Analyze pressure-contraction behaviour.

- Analyze pressure-force behaviour.

- Analyze radial expansion.

- Calculate hysteresis.

- Compare initial and final dimensions.

- Analyze elastic recovery.

- Analyze structural degradation.

- Analyze cyclic degradation.

- Identify failure locations.

- Identify failure modes.

Final Results

| Parameter | Initial Condition | Final Condition | Change |
| --- | --- | --- | --- |
| Actuator length | L₀ | Lf | ΔL |
| Diameter | D₀ | Df | ΔD |
| Maximum contraction | Initial | Final | Change |
| Maximum force | Initial | Final | Change |
| Leakage condition | Initial | Final | Change |
| Braid condition | Initial | Final | Observation |
| Structural condition | Initial | Final | Observation |

Output

- Complete test database

- Final pressure-contraction curves

- Final force-pressure curves

- Hysteresis analysis

- Elastic integrity assessment

- Structural integrity assessment

- Endurance assessment

- Failure mode analysis

- Final validated actuator specification

Summary of Phase-Wise Programme

| Phase | Duration | Activity |
| --- | --- | --- |
| 1 | July to August | Literature study, theoretical analysis, calculations and CAD development, Completed |
| 2 | September, Week 1 to 2 | Test-rig design and commissioning |
| 3 | September, Week 3 to 4 | Baseline characterization and initial pressure testing |
| 4 | October, Week 1 to 3 | Elastic integrity validation |
| 5 | October, Week 4 to November, Week 2 | Structural integrity validation |
| 6 | November, Week 3 to 4 | Force-contraction characterization |
| 7 | December, Week 1 to 2 | Cyclic and endurance testing |
| 8 | December, Week 3 | Final validation and proof testing |
| 9 | December, Week 4 | Data analysis and final documentation |