Derivation for soft actuator play my:

- In a first approach toward the model of a McKibben type PAM, losses are not considered and thus, the gas enters the PAM and expands, generating a traction force and a work. To generate the volume increase, the gas performs a work input (Win) that can be described.

dWin = ∫(P − Pex )dli.dSi

= (P − Pex) ∫dli.dSi

dWin = P’dV

Where, Pex = External Force,

P  = Absolute Pressure

P’ = Relative Pressure

- As the pressure performs work by volume expansion and it is assumed that the system has no loss, all the work is converted into traction and can be expressed by Wout,

dWout = -FdL

Where, F is the axial tension and dL is the displacement.

The Conservation of Energy gives:

dWin = dWout

P’dV = -FdL

F = - (P’ dV)/dL ........................ (1)

- With the strength defined in terms of the volume change, just missing define the volume variation permitted by the mesh. The variation stops when volume reaches its maximum. At this moment, the muscle reaches the largest contraction.

- Observing Fig.3, it is possible to make the length (L) and diameter (D) as a function of the angle (θ). This angle is one formed between the mesh and the axial length of the muscle. The length of fiber can be named b and n is the number of turns of the fiber around the muscle. Using b and n, it is possible to set the length and the diameter of the muscle.

cosθ= Lb

L = b cos…….……………………..…………………………… (2)

sinθ =Dnπb

D= bsinθnπ …………….…….……………………………..……. (3)

Making the Volume equation as function of the diameter and D & length L.

V= 14*πD2L

By substituting D & L in eq 4

We get, 										V= b34πn2 *sin2θ*cosθ……………………………….. (5)

Diff. eq (2) & eq (5) w. r. t.

dLdθ=-bsinθ ……………………………..….…..……………………. (6)

dVdθ= b34πn2 *[3cos2θ -1] …….………………………………. (7)

Substitute the value of eq (6) & eq (7) in eq (1)

F=-P'* dVdL=-P'* dVdθdLdθ

After substituting and rearranging, we get:

F= P'D2 (3cos2θ -1)4πn2 ……………………............................... (8)

D= bsinθnπ

Dsinθ= bnπ

By substituting the value of D & trigonometric relation in eq (8)

F= π4 P'D2* (3cos2θ -1)sin2θ ……………………………….….. (9)

Adopting contraction (ε) and applying Eq.9, it is possible to reach to required formulation (Daerden and Lefeber, 2000).

D= bsinθnπ ………………………………… After applying pressure

D0= bsinθ0nπ ……….......................... Before applying pressure

D = D0 ………………………………..……. Basic Interpretation

So, we can replace D & sin with D0 & sin0 in eq (9)

cosθ= Lb

L = b cosθ

L0= b cosθ0

LL0= cosθcosθ0

By Rearranging,

cosθ= LL0 cosθ0 ……………………………..……………. (10)

Where, Contraction Factor ()

ε= L0-LL0=1 - LL0

1-ε= LL0

Put this value in eq (10)

cosθ=(1-ε) * cosθ0

Substitute cos, D = D0 & sin = sin0 in eq (9)

F= π4 P'D2* (3(1-ε)2 * cos2θ0-1)sin2θ0